## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest-cost technically compliant kit

- The quantities mentioned below are required unless there is a variation within a specified range with proper justification. The LRC reserves the right to increase or decrease the quantities.

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid, rather than the total price without go back to the supplier. The unit price must be provided with no more than two decimal places.

- Only one option is allowed for biding

- Datasheet + Picture for each item must be submitted with the bid and labelled reference to each item number

LOT 1: Fire Extinguisher

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT rated at 11%, but inclusive Labor, installation, Bank Transfer Fees, and Delivery fees | VAT (%) | Unit Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Total Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Lead time of delivery for any quantity from the date of each purchase order |
| 1.1 | Fire ext CO2 3KG | PIECE | 150 |  |  |  |  |  |
| 1.2 | Fire ext CO2 5KG | PIECE | 150 |  |  |  |  |  |
| 1.3 | Fire ext powder 1 kg | PIECE | 50 |  |  |  |  |  |
| 1.4 | Fire ext powder 2 kg | PIECE | 50 |  |  |  |  |  |
| 1.5 | Fire ext powder 6kg | PIECE | 200 |  |  |  |  |  |
| 1.6 | Fire ext powder 9kg | PIECE | 100 |  |  |  |  |  |
| 1.7 | Fire ext powder 12 kg | PIECE | 50 |  |  |  |  |  |
| 1.8 | Fire ext powder 30 kg | PIECE | 20 |  |  |  |  |  |
| 1.9 | Automatic fire ext 6 KG | PIECE | 100 |  |  |  |  |  |
| 1.10 | Automatic fire ext 12 KG | PIECE | 50 |  |  |  |  |  |
|  | Total for Lot 1 |  |  |  |  |  |  |  |

LOT 2: MEGAPHONE

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT rated at 11%, but inclusive Labor, installation, Bank Transfer Fees, and Delivery fees | VAT (%) | Unit Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Total Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Lead time of delivery for any quantity from the date of each purchase order |
| 2.1 | Megaphone (high Power) | PIECE | 100 |  |  |  |  |  |

LOT 3: SAFETY SELF PROTECTION

| LRC To Complete |  |  | Bidder to Complete |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT rated at 11%, but inclusive Labor, installation, Bank Transfer Fees, and Delivery fees | VAT (%) | Unit Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Total Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Lead time of delivery for any quantity from the date of each purchase order |
| 3.1 | Colored vest | PIECE | 700 |  |  |  |  |  |
| 3.2 | Fire Safety box | BOX | 100 |  |  |  |  |  |
| 3.3 | Fire blanket | PIECE | 100 |  |  |  |  |  |
| 3.4 | Fire Face mask | PIECE | 150 |  |  |  |  |  |
| 3.5 | Safety helmet | PIECE | 150 |  |  |  |  |  |
| 3.6 | Safety Gloves | PAIR | 200 |  |  |  |  |  |
| 3.7 | Firefighting Glasses | PIECE | 100 |  |  |  |  |  |
| 3.8 | Fire Boots | PAIR | 100 |  |  |  |  |  |
| 3.9 | Fire Fighting Fire suit & Trouser | SET | 50 |  |  |  |  |  |
| 3.10 | Wildland Fire Helmet | PIECE | 100 |  |  |  |  |  |
| 3.11 | Ear, neck and Face Protector for Wildland Helmets | PIECE | 100 |  |  |  |  |  |
| 3.12 | Wildland Firefighting Gloves | PIECE | 100 |  |  |  |  |  |
| 3.13 | Safety Goggle | PIECE | 100 |  |  |  |  |  |
| 3.14 | Short sleeves fire fighting shirt | PIECE | 100 |  |  |  |  |  |
| 3.15 | Wildfire standard suit | PIECE | 100 |  |  |  |  |  |
| 3.16 | Wildfire fighting Boots | PIECE | 20 |  |  |  |  |  |
| 3.17 | Air breathing apparatus | PIECE | 30 |  |  |  |  |  |
| 3.18 | Collapsible Backpack Firefighting Pump with Standard Piston Pump | PIECE | 100 |  |  |  |  |  |
|  | Total for Lot 3 |  |  |  |  |  |  |  |

LOT 4: EMERGENCY EQUIPMENT

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT rated at 11%, but inclusive Labor, installation, Bank Transfer Fees, and Delivery fees | VAT (%) | Unit Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Total Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Lead time of delivery for any quantity from the date of each purchase order |
| 4.1 | Electric siren/Emergency Siren | PIECE | 40 |  |  |  |  |  |
| 4.2 | Assembly point (Plastic) | PIECE | 100 |  |  |  |  |  |
| 4.3 | Elevator Sign | PIECE | 50 |  |  |  |  |  |
| 4.4 | Reflective Sign | PIECE | 100 |  |  |  |  |  |
| 4.5 | Chopper Axe | PIECE | 100 |  |  |  |  |  |
| 4.6 | Heavy duty hoe | PIECE | 100 |  |  |  |  |  |
| 4.7 | Firefighting Shovel | PIECE | 100 |  |  |  |  |  |
| 4.8 | Smoke detector | PIECE | 50 |  |  |  |  |  |
| 4.9 | Fire alarm system | System | 5 |  |  |  |  |  |
| 4.10 | Break Glass | PIECE | 100 |  |  |  |  |  |
| 4.11 | Cables and PVC 50 M | PIECE | 100 |  |  |  |  |  |
| 4.12 | Cables and PVC 100 M | PIECE | 100 |  |  |  |  |  |
| 4.13 | Hoze 1 | PIECE | 20 |  |  |  |  |  |
| 4.14 | Hoze 2 | PIECE | 20 |  |  |  |  |  |
| 4.15 | Nozzle | PIECE | 20 |  |  |  |  |  |
| 4.16 | Gasoline saws | PIECE | 20 |  |  |  |  |  |
| 4.17 | Pulaski Axe | PIECE | 40 |  |  |  |  |  |
| 4.18 | Firefighting Axe | PIECE | 40 |  |  |  |  |  |
| 4.19 | Folding saw | PIECE | 40 |  |  |  |  |  |
| 4.20 | Metric Diameter Tape | PIECE | 50 |  |  |  |  |  |
| 4.21 | Hydration Water Pack | PIECE | 100 |  |  |  |  |  |
| 4.22 | Brush cutter | PIECE | 100 |  |  |  |  |  |
| 4.23 | مشط/Metal rake | PIECE | 100 |  |  |  |  |  |
|  | Total for Lot 4 |  |  |  |  |  |  |  |

LOT 5: RIFILLING

- LRC do not currently have information about the types and capacities (KG) of the existing fire extinguishers installed in the schools and community sites. Therefore, suppliers are requested to take this into consideration when preparing their offers and, if needed, include site verification in their scope.

- The submitted prices must be inclusive of transportation and installation costs covering all Lebanese regions, with no additional charges to be applied later for delivery or on-site installation.

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT rated at 11%, but inclusive Labor, installation, Bank Transfer Fees, and Delivery fees | VAT (%) | Unit Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Total Price in USD, inclusive VAT, Labor, installation, Bank Transfer Fees, and Delivery and all other fees | Lead time of delivery for any quantity from the date of each purchase order |
| 5.1 | POWDER REFILLING | KG | 2800 |  |  |  |  |  |
| 5.2 | CO2 REFILLING | KG | 700 |  |  |  |  |  |

## ANNEX 3: DETAILED SPECIFICATIONS

| Lot 1 | FIRE EXTIGUISHER |  |  |
| --- | --- | --- | --- |
| ITEM # | ITEM/MILESTONE REQUIRED | UOM | DESCRIPTION |
| 1.1 | Fire ext CO2 3KG | PIECE | Charge - CO2 Gas ¦ Capacity - 3kg ¦ Rating - 21B Minimum Effective Discharge Time - 8 Seconds (Minimum) Bulk Range Discharge - 2 Meter (Minimum) Cylinder Testing Pressure - 250 bar Gas Storage Pressure - 50kg/cm2 approx. Filling Density - 0.667 Max. Working Temperature - -30°C to +55°C Weight of one Charged Extinguisher - 17Kg (approx.) Standard Specificat ion - IS 15683:2006 Suitable for - B and C Class Fire |
| 1.2 | Fire ext CO2 5KG | PIECE | Charge - CO2 Gas ¦ Capacity - 5kg ¦ Rating - 13B Minimum Effective Discharge Time - 8 Seconds (Minimum) Bulk Range Discharge - 1 Meter (Minimum) Cylinder Testing Pressure - 250 bar Gas Storage Pressure - 50kg/cm2 approx. Filling Density - 0.667 Max. Working Temperature - -30°C to +55°C Weight of one Charged Extinguisher - 12.5Kg (approx.) Standard Specification - IS 15683:2006 Suitable for - B and C Class Fire |
| 1.3 | Fire ext powder 1 kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.4 | Fire ext powder 2 kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.5 | Fire ext powder 6kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.6 | Fire ext powder 9kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.7 | Fire ext powder 12 kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.8 | Fire ext powder 30 kg | PIECE | Certification: UL Listed Standards: ANSI / UL 299, ANSI / UL 711 Fire rating: Class A, B and C fire Temperature: - 40°C to 49°C Pressure: 195 psi Chemical Agent: ABC powder, ammonium phosphate-based chemicals Hose: Rubber Cylinder: Deep drawn steel Cylinder valve: Aluminum, with pressure gauge and SS handle Wall hook or wall bracket should be included |
| 1.9 | Automatic fire ext 6 KG | PIECE | Certification: UL Listed Standards: ANSI / UL 154, ANSI / UL 711 Fire rating: Class B and C fire Temperature: - 30°C to 49°C Discharge range: 4 ft ~ 8 ft Discharge time: 6-10 sec Agent: CO2 (Carbon dioxide) Hose: Rubber Cylinder: Aluminum Cylinder valve: Brass, with SS handle Wall hook or wall bracket should be included |
| 1.10 | Automatic fire ext 12 KG | PIECE | Certification: UL Listed Standards: ANSI / UL 154, ANSI / UL 711 Fire rating: Class B and C fire Temperature: - 30°C to 49°C Discharge range: 4 ft ~ 8 ft Discharge time: 6-10 sec Agent: CO2 (Carbon dioxide) Hose: Rubber Cylinder: Aluminum Cylinder valve: Brass, with SS handle Wall hook or wall bracket should be included |
| Lot 2 | MEGAPHONE |  |  |
| ITEM # | ITEM/MILESTONE REQUIRED |  | DESCRIPTION |
| 2.1 | Megaphone (high Power) | PIECE | Output 50 Watts '- Siren '- Volume control '- Detachable Megaphone '- Effective distance :400 m '- 1.5VX8 “D size”battery '- Horn diameter 240 mm '- 10 seconds for recording Bulb release Temperature 60-86* |
| LOT 3 | SAFETY SELF PROTECTION |  |  |
| ITEM # | ITEM/MILESTONE REQUIRED |  | DESCRIPTION |
| 3.1 | Colored vest | PIECE | All sizes. ¦ Reflective Glass Beads |
| 3.2 | Fire Safety box | BOX | Color:Flame Red- Powder coated Material: 0.9 Zinc Anneal Sheet Metal Lock:Cam Lock or emergency Break Glass With Glass Window Dimention: 710 x 320 x 220 (estimated) |
| 3.3 | Fire blanket | PIECE | Glass fibre fabric independently tested and certified to BS 476 parts 4 and 7 Blanket made from woven glass fibre fabric (totally asbestos-free) Containers can be wall-mounted Easy and quick withdrawal of blanket from container English and/or other language instruction labels available Clear pictorial instructions |
| 3.4 | Fire Face mask | PIECE | Item Name: hig+A4:B12h quality double respirator mask/gas mask Model No: CH-M002 Mask material: PVC (polyester optional) Cartridge material: activated carbon Strap: Easily adjustable Style: double active-carbon can or single cartridge optional Size & weight: optional Application: Great for filtering small dusts, mists, metallic fumes, spray paint, chemicals, and small flying particles of LOW toxicit. Standard: CE... |
| 3.5 | Firefighter helmet | PIECE | outer shell material: High temperature polytherimide (PEI) plastic with single reinforcement at tope ; Face shield material:polyetherimide（PEI） plastic Face shield material:polyetherimide（PEI） plastic Neck protector: two layer. outer layer is aramid fiber, inner layer is fire-retardant cotton. Head Shell Heat resistant: 260°C neck and ear coverage and protection All-round head protection. Free adjustment of the helmet. Adjustable suspension system Chin strap High temperature, heat resistant, anti-impact, anti abrasion, shock absorption and fire proof. Color:yellow/red |
| 3.6 | Safety Gloves | PAIR | The gloves have four layer made of aramid outer layer: adopts the high strength fabric with flame retardant anti - acid and alkali, anti - oil, anti - static, etc.. Waterproof layer: waterproof and moisture permeable fabric. Insulation layer: using a heat insulation, flame retardant, thermal radiation properties of fabrics. Comfort layer: comfortable fabric with flame retardant properties. Style: five finger Color: navy blue Applications: suitable for fire protection, the opponent of the wrist, to prevent scratches and cut |
| 3.7 | Fire Safety Glasses | PIECE | Made of asbestos,can protect the fire, widely used in fire-fighting,oil,coal, and mining industries ect. Specification: 1)Igood tensile strength and good stays and anticorrosive,performance flameretardant, prevent water resistance, resistance to grind, wet through every hot, prevent static electricity, comfortable. 2)Length > 30 cm 3)Thickness 2.0mm |
| 3.8 | Fire Boots | PAIR | Outer sole:made in one-piece slip-resistant rubber Outer materail:PMIA three-in-one aramid fabric Inner lining fabric:flame retardant treatment. Thermal barrier:aramid fiber felt Midsole: steel Toecap: steel In the front of the barrel, it has a tibia protection in yellow,made of rubber Size :38-48 have resistance to: abrasion, perforation and heat penetration, has additional reinforcements on the instep and sides |
| 3.9 | Fire Fighting Fire suit & Trouser | SET | Outer materail:PMIA three-in-one aramid fabric Moisture barrier layer:Aramid non woven+PTFE membrane Thermal barrier:aramid fiber felt 3M reflective stap color: navy blue Color aberration: 4 grade Fire resistance performance Damaged length: ≤10cm, Continued burning time ≤2s, No drop phenomenon Breaking strength: warp and weft≥650N |
| 3.10 | Wildland Fire Helmet | Pcs | Material: Super plastic High temperature 400 degrees no deformation Two side lamp holders High quality reflective strip Shock-absorbing lining C-clamp on the side |
| 3.11 | Ear, neck and Face Protector for Wildland Helmets | Pcs | Martial: Flame retardant fabric |
| 3.12 | Wildland Firefighting Gloves | Pcs | Goatskin Leather manufacturing Palm wear resistance 2000 Cutting force 40N Tearing force 131N Piercing force 68N. The hydrostatic pressure is not less than 7KPa, & the overall thermal protection is 30.4 CAL/cm. Material: Flame retardant fabric |
| 3.13 | Safety Goggle | Pcs | Lens PC material Heat resistant No deformation VEX anti-fog coating |
| 3.14 | Short sleeves fire fighting shirt | Pcs | 4.3 oz 100% Fine Jersey cotton construction short sleeve t-shirt. Softest and Smoothest. Durable rib neckband. |
| 3.15 | Wildfire standard suit | Pcs | Color: Yellow, Black or Red Flame retardant performance Tensile strength Tested Tear strength Tested Thermal stability Tested Formaldehyde content Tested pH value Tested Water resistance Fastness in washing Dry friction resistance, the color fastness to perspiration, Sewing thread Sewing thread thermal stability. Reflective tape back reflection coefficient values on both legs and waist. Standards for wild land firefighting protective clothing and UL classified. Accepted by NFPA or any equivalent source in the country of origin. |
| 3.16 | Wildfire fighting Boots | Pcs | Top layer trim skin 1.6 mm Flame retardant Kevlar Rubber sheet Steel plate Reflective cloth shoelace retardant |
| 3.17 | Air breathing apparatus | PIECE | Cylinder: 6.8L Cylinder size: 157x520mm Inside material: 6061 aluminium alloy Out material: carbon fiber + epoxy resin+fire resistant Agent: breathing air Filling pressure: 30Mpa Test pressure: 50Mpa Thread valve: M18X1.5-6H Using life: 15 years Mask: clip-on Respiratory depressor: safety & stability 0.7Mpa Backpack: fire resistant material Package:Black or orange blow case |
| 3.18 | Collapsible Backpack Firefighting Pump with Standard Piston Pump/Knapsack water mist | Pcs | Capacity: 20 L Material: PVC net clamping cloth Brass Flit style spray gun 48.5x40.5x30.5 cm/12kgs Spray jet length 8 meter |
| LOT 4 | EMERGENCY EQUIPMENT |  |  |
| ITEM # | ITEM/MILESTONE REQUIRED |  | DESCRIPTION |
| 4.1 | Electric siren | PIECE | Electric sirens with evacuation alarm tones at multiple sites Provide and install necessary components (siren, cable, bell push) Ensure the system emits a clear evacuation sound (emergency siren) different from regular school bells Type: Motor siren Diameter: 80–100 mm Material: Steel & ABS Color: Red Power: 220V AC, 10–40W Sound Output: 90–200 dB Mounting: Wall-mounted Protection: IP44 Speed: 10,000 rev/sec Size: 12 x 16 x 19 cm Weight: 2 kg Model: MS-290 Certification: Not required Accessories & Installation Cable: 20m per site, 1.5mm thickness Cable cache: 1cm Bell push: 220V, 120dB, with evacuation sound tone |
| 4.2 | Assembly point (Plastic) | PIECE | Rigid plastic for wall mounting Dimensions: 60x40 cm Color: Green and White Language: Arabic and English |
| 4.3 | Elevator Sign | Pcs | 15CM*17CM - White plastic - Green vinyl |
| 4.4 | Reflective Sign | Pcs | 17CM*27CM - Plexi - Green Vinyl reflective & white vinyl Reflective |
| 4.5 | Chopper Axe | PIECE | Head: carbon steel Handle Material: FiberGlass/Wood Process: drop forged, polished or spray painted Weight:600g Size:38cm Color:Black Type: Flat Backed Axe |
| 4.6 | Heavy duty hoe | Pcs | Material: Rail steel Weight 2.2 to 2.5 Kg Width 15 cm Length 24 cm With heavy duty wooden Handle: Weight 0.8-1 Kg Length 135 cm, diameter 4 to 6cm |
| 4.7 | Firefighting Shovel | Pcs | Carbon steel blade 10-7/8” long 8-7/8” wide 41” Ash handle with solid shank Overall length: 51-7/8” |
| 4.8 | Smoke detector | PIECE | LAUT 85 Dezibel Alarm, 220V, or equivalent |
| 4.9 | Fire alarm | System | 4 ZONE - Coventional CE - COFEM Brand or equivalent /24V 8 ZONE - Coventional CE - COFEM Brand or equivalent / 24V Smoke detector - Coventional -24V - CE - COFEM Brand or equivalent Break glass - Coventional - 24V - CE - COFEM Brand or equivalent Siren - Conventional - 24V - CE - COFEM Brand or equivalent Installation: PVC /M - 16MM and Fire cable /M: 1.5MM |
| 4.10 | Break Glass | Pcs | Power Supply 24 - 35V with polarity Standby Current 1 mA Alarm Current 5 mA Activation Signal Red Light Remote Indicator Output No Humidity 20 - 95% RH Temperature -10ºC +50ºC Standard EN 54-11 IP Protection IP 50 • Easily resettable call point by pushing yellow button on the front side • Transparent protector cover to avoid accidental false alarms. • Self-identified element in the fire detection algorithmic and addressable • Communication with the control panel is indicated by a single flash of the LED • Immediate visual recognition of alarm status by the permanent activation of the LED, and the trigger of the yellow tab on the lower side of the activation face. • According to EN 54-11 Standard, and CE mark according to the European Regulation of Construction Products (UE) N°305/2011. |
| 4.11 | Fire cable | m | 1.5MM - 2 cores - Red cable - Flame test 750 deghree - 100% Copper |
| 4.12 | PVC tube | m | 16MM - Not stabilized - Resistent to flame - Grey colour |
| 4.13 | Hoze 1 | Pcs | Size: 1''x30M Connection: with BS aluminum coupling Material: *Jacket: polyester *Lining: PU Working pressure: 10bar Color: white |
| 4.14 | Hoze 2 | Pcs | HOSE REEL /PVC /20M/13BAR Size: 1.5''x20M Color: red Fire Hose Lining: PVC Fire Hose size: 1.5 Inch Working Pressure: 13 Bar Bursting Pressure: 24-72 Bar Length: 20m Color: red Usage: firefighting/ irrigation/agriculture Coupling size: 2.5 inch inlet, and 2.5 inch outlet Coupling: aluminum |
| 4.15 | Nozzle | Pcs | Inlet size: 65mm, however could be changed to 50mm, 40mm according to customer's requirement Rated pressure: 0.6Mpa Rated flow: 500L/min Pressure range: 3.5-16 bar Flow regulation: 2.5-4-5-6.5-8L/S Connect style: Selectable (British, German, Japanese, Russian, French, etc.) Basis function: Tight straight stream and wide dense fog Materials: Aluminum alloy (TS treatment) Surface: Anodized anti-corrosion treatment |
| 4.16 | Gasoline saws | Pcs | Displacement: 58cc RPM:8000/min Chainsaw: 18" 20" Max output(kw): 2.3 kw Engine Type: 2 STROKE Fuel tank Capacity: 0.55L Carburettor Type: Diaphragm Clutch System: Automatic Centrifugal Start System: Recoil Starter Weight: 5.8KG |
| 4.17 | Pulaski Axe | Pcs | Pulaski Axe with fiberglass handle. Axe Head weight approx. 1.6 kg, made from forged carbon steel. Steel cutting part heated 45-55HRC |
| 4.18 | Firefighting Axe | Pcs | Material: Carbon steel HRC: 48-58 Weight: approx 3kg Size: approx. 93x26x11.4 cm Red glue processing doesn't turn around Wood handle with ergonomic design |
| 4.19 | Folding saw | Pcs | Blade for 65 manganese steel With lock folding Open 43 cm Fold 18 cm With rubber handle 3 sides saw teeth |
| 4.20 | Metric Diameter Tape | Pcs | 50m pvc meter waterproof, markings for meters, centimeters, and millimeters |
| 4.21 | Hydration Water Pack | Pcs | Water capacity 2L Waterproof and tear resistant Size 39x28x6.6 cm Weight 0.54 kg Quick drinking water system Chest adjustable buckle 88-128 cm |
| 4.22 | Brush cutter | Pcs | Displacement: 42.7cc Max output(kw): 1.27 Engine Type: 2 STROKE Egine Specification: Fuel tank Capacity: 1.1L Carburettor Type: Diaphragm Start System: Recoil Starter Clutch System: Automatic Centrifugal Shaft ASS’Y Specification: Main shaft Spec.: 8mmX1530mmX9T Pipe Spec: 26X2.0X1480mm |
| 4.23 | مشط/Metal rake | Pcs | Steel rake rectangular 18 teeth, 41x11cm approximately Sharp rake Wooden handle |
| LOT 5 | REFILLING |  |  |
| 5.1 | POWDER REFILLING | KG | Chemical Agent: ABC powder, ammonium phosphate-based chemicals 1 KG QTY: 40 2 KG QTY: 40 6 KG QTY : 1000 9 KG QTY: 400 12 KG QTY : 500 30 KG QTY: 20 4.5 Kg Qty: 100 AUTO 6 KG QTY: 400 AUTO 12 KG QTY:300 |
| 5.2 | CO2 REFILLING | KG | Gas Storage Pressure - 50kg/cm2 approx. 3 KG QTY: 350 5 KG QTY: 350 |