## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest-cost technically compliant bid per item

- Under this Framework Agreement, the quantities indicated are estimates only and may vary depending on LRC operational requirements. Purchases will be executed through the issuance of Purchase Orders on an as-needed basis during the validity period of the agreement. LRC does not guarantee the procurement of the full estimated quantities.

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid, rather than the total price without go back to the supplier. The unit price must be provided with no more than two decimal places.

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item # | Item/Milestone Required | Unit of Measurement (UOM) | Estimated Quantity | Unit Price in USD Exclusive of VAT, but inclusive of labor, bank transfer fees, and delivery fees. | VAT (%) | Unit Price in USD Inclusive of VAT, labor fees, bank transfer fees, and delivery fees. | Total Price in USD Inclusive of VAT, labor fees, bank transfer fees, and delivery fees. | Lead time of delivery (in calendar days) from the date of each Purchase Order. |
| 1 | Laptop High Specs | Pcs | 14 |  |  |  |  |  |
| 2 | Portable high-capacity power bank, 80000 mAh – 100W USB-C | Piece | 28 |  |  |  |  |  |
| 3 | Combo Keyboard and Mouse | Piece | 21 |  |  |  |  |  |
| 4 | Interactive Board | Piece | 5 |  |  |  |  |  |
| 5 | Whiteboard Screen + Tripod | Piece | 1 |  |  |  |  |  |
| 6 | Laptop Basic | Piece | 15 |  |  |  |  |  |
| 7 | External Monitor 24 ‘’ | Piece | 10 |  |  |  |  |  |
| 8 | Wireless Mousse | Piece | 35 |  |  |  |  |  |
| 9 | Smart TV 65 Inch | Piece | 7 |  |  |  |  |  |
| 10 | Satellite Phones | Piece | 3 |  |  |  |  |  |
| 11 | Laser printers | Piece | 17 |  |  |  |  |  |
| 12 | Video conferencing Systems | Piece | 3 |  |  |  |  |  |
| 13 | Speakers | Piece | 4 |  |  |  |  |  |
| 14 | Mobile Stand for interactive Board | Piece | 2 |  |  |  |  |  |
| 15 | Laptop Standard Specs | Piece | 5 |  |  |  |  |  |
| 16 | Laptop Medium Specs | Piece | 8 |  |  |  |  |  |
| 17 | Tablets Standard , with Adapter, cover, and screen protection | Piece | 58 |  |  |  |  |  |
| 18 | Tablet High Specs , with Adapter, cover, and screen protection | Piece | 6 |  |  |  |  |  |
| 19 | External Monitor 27 ‘’ | Piece | 9 |  |  |  |  |  |
| 20 | Portable monitor | Piece | 10 |  |  |  |  |  |
| 21 | Mobile Phone, with Adapter, cover, and screen protection | Piece | 22 |  |  |  |  |  |
| 22 | Wireless access Points | Piece | 4 |  |  |  |  |  |
| 23 | LTE/4G/5G Routers | Piece | 27 |  |  |  |  |  |
| 24 | Satellite Internet Terminals (StarLink) | Piece | 4 |  |  |  |  |  |
| 25 | IP Phone | Piece | 15 |  |  |  |  |  |
| 26 | Grandstream PBX | Piece | 1 |  |  |  |  |  |
| 27 | Shredder | Piece | 2 |  |  |  |  |  |
| 28 | HDMI Cable 1.5 m | Piece | 7 |  |  |  |  |  |
| 29 | HDMI Cable 3 m | Piece | 7 |  |  |  |  |  |
| 30 | HDMI Cable 5 m | Piece | 6 |  |  |  |  |  |
| 31 | Type C to 4 HDMI Splitter | Piece | 5 |  |  |  |  |  |
| 32 | Portable Projector | Piece | 6 |  |  |  |  |  |
| 33 | External hard drives | Piece | 10 |  |  |  |  |  |
| 34 | GIS Workstations | Piece | 3 |  |  |  |  |  |
| 35 | Docking Station | Piece | 2 |  |  |  |  |  |
| 36 | Webcam camera | Piece | 2 |  |  |  |  |  |
| 37 | Portable Printer with Battery | Piece | 2 |  |  |  |  |  |
| 38 | Laptop Backpack | Piece | 8 |  |  |  |  |  |
| 39 | MousePad | Piece | 4 |  |  |  |  |  |
| 40 | Ultra wide monitor 34 inch | Piece | 4 |  |  |  |  |  |
| 41 | Powerbank 250 W | Piece | 4 |  |  |  |  |  |
| 42 | Drum for Konica Minolta bizhub C258 Black Drum Unit | Piece | 4 |  |  |  |  |  |
| 43 | Drum for Konica Minolta bizhub C258 Color Drum Units (C / M / Y) | Piece | 4 |  |  |  |  |  |
| 44 | Waste Toner for Konica Minolta bizhub C258 | Piece | 2 |  |  |  |  |  |
| 45 | Plotter INK for HP DesignJet T930 | Set | 2 |  |  |  |  |  |
| 46 | Toner black for Konica Minolta C258 | Piece | 2 |  |  |  |  |  |
| 47 | Toner cyan for Konica Minolta C258 | Piece | 2 |  |  |  |  |  |
| 48 | Toner magenta for Konica Minolta C258 | Piece | 2 |  |  |  |  |  |
| 49 | Toner Yellow for Konica Minolta C258 | Piece | 2 |  |  |  |  |  |
| 50 | Drum (black and colored) for Sharp DX-2500 | Set | 2 |  |  |  |  |  |
| 51 | Toner black, for Sharp DX-2500 | Piece | 2 |  |  |  |  |  |
| 52 | Toner cyan, for Sharp DX-2500 | Piece | 2 |  |  |  |  |  |
| 53 | Toner magenta, for Sharp DX-2500 | Piece | 2 |  |  |  |  |  |
| 54 | Toner Yellow, for Sharp DX-2500 | Piece | 2 |  |  |  |  |  |
| 55 | Toner for Cannon Pixma black | Piece | 8 |  |  |  |  |  |
| 56 | Toner for Cannon Pixma Colored | Piece | 2 |  |  |  |  |  |
| 57 | Microsoft Windows 11 Enterprise | Lic | 30 |  |  |  |  |  |
| 58 | Microsoft Entra ID P2 | Lic | 30 |  |  |  |  |  |
| 59 | Microsoft Defender for Endpoint | Lic | 30 |  |  |  |  |  |
| 60 | Microsoft 365 E3 | Lic | 30 |  |  |  |  |  |
| 61 | Microsoft 365 E5 | Lic | 30 |  |  |  |  |  |
| 62 | Power BI | Lic | 10 |  |  |  |  |  |
| 63 | Adobe Acrobat Pro | Lic | 2 |  |  |  |  |  |
| 64 | Copilot Studio | Lic | 1 |  |  |  |  |  |
| 65 | Canva Business | Lic | 3 |  |  |  |  |  |
| 66 | Photoshop beta 2026 | Lic | 2 |  |  |  |  |  |
| 67 | Google Flow | Lic | 1 |  |  |  |  |  |
| 68 | AutoCAD 2026-Architecture | Lic | 2 |  |  |  |  |  |
| 69 | Sketchup | Lic | 2 |  |  |  |  |  |
| 70 | Lumion 2026 software | Lic | 1 |  |  |  |  |  |

## ANNEX 3: DETAILED SPECIFICATIONS

| ITEM # | ITEM NAME | ITEM Specification | QTY / UOM |
| --- | --- | --- | --- |
| 1 | Laptop High Specs | Processor: CPU must be Intel Core Ultra Ultra 9-185H class CPU Architecture: Processor shall be Intel Core Ultra hybrid architecture with AI / NPU acceleration Chipset Platform: must be Intel integrated SoC platform Graphics: Graphics must be NVIDIA RTX 500 Ada Generation or Intel Arc integrated GPU (workstation class) RAM: Installed memory shall be 16 GB DDR5 5600 MHz minimum Memory Slots: Must support DDR5 SODIMM expansion up to ≥ 96 GB Storage: Internal storage shall be 512 GB PCIe Gen4 NVMe SSD minimum Storage Interface: Storage must be M.2 PCIe 4.0 NVMe with Opal encryption support Additional Storage: Platform must support dual M.2 NVMe slots Display Size: Display must be exactly 16.0-inch Display Resolution: Resolution must be 1920 × 1200 (WUXGA) minimum Display Ratio: Aspect ratio shall be 16:10 Display Panel: Panel must be IPS anti-glare, ≥ 300 nits, Eyesafe / low blue light class Optional High-Resolution Support: Platform shall support 2560×1600 / OLED / high-resolution panel option Refresh Rate: Display must support 60 Hz minimum Wireless: Must support Wi-Fi 6E (802.11ax) Bluetooth: Must support Bluetooth 5.3 or later Ethernet: Must include RJ-45 LAN port integrated USB Ports: Must include USB-A + USB-C + HDMI + Audio combo USB-C Standard USB-C must support Thunderbolt 4 / USB4 + DisplayPort + Power Delivery HDMI Must support HDMI digital output External Display Support: Must support up to 4 displays via HDMI / Thunderbolt / USB-C Camera: Integrated webcam must be FHD 1080p with privacy shutter / IR option Audio: Stereo speakers with digital audio enhancement Keyboard: Full keyboard with numeric keypad required Security: Must support TPM 2.0 + fingerprint reader + IR authentication support Operating System: Must be delivered with Windows 11 Pro 64-bit Battery: Integrated battery must be ≥ 75 Wh class Power Adapter: Must include USB-C fast charging adapter Certification: Platform must support ISV / workstation-class certification | 14 Pcs |
| 2 | Portable high-capacity power bank, 80000 mAh – 100W USB-C | Battery Capacity: Internal battery must be 80000 mAh (≈256 Wh) Battery Type: Battery shall be Lithium Iron Phosphate / Lithium high-capacity battery pack USB-C Output 1: Must support 100W USB-C Power Delivery output USB-C Output 2: Must support 65W USB-C Power Delivery output USB-C Output 3: Must support 15W USB-C output USB-A Outputs : Must include 2 × USB-A ports with 22.5W fast charging support Total Outputs Must support minimum 5 output ports simultaneously Input Charging: Must support USB-C + Lightning + DC input charging Fast Charging: Must support Power Delivery + Quick Charge fast charging protocols Simultaneous Charging: Must support charging multiple devices at the same time without power drop Supported Devices: Must support laptop, tablet, smartphone, camera, router, drone charging Display: Must include digital LED battery status display Lighting: Must include integrated LED torch with multiple modes Protection: Must include over-charge, over-current, short-circuit, temperature protection Output Voltage: Must support 5V / 9V / 12V / 15V / 20V output profiles Output Power: Maximum output power must be 100W Recharging Modes: Must support multi-source recharging including DC / USB-C / Lightning Portable Use: Must support portable power station operation without external supply Housing: Must include integrated handle / rugged portable design Safety: Must include intelligent power management and battery protection circuit Application: Must be suitable for field work, outdoor use, emergency power, ICT equipment backup | 28 Pcs |
| 3 | Combo Keyboard and Mouse | Keyboard - Full-size layout with Arabic + English characters, including numeric keypad. - Quiet keys with familiar spacing for comfortable daily typing. - Spill-resistant design for extra durability. - Built-in palm rest and adjustable tilt legs for wrist comfort. - Multimedia shortcut keys for quick control of volume, media, and calculator. - Wireless connection via a 2.4 GHz USB receiver with stable, encrypted transmission. - Battery life up to 36 months depending on usage. Mouse - Contoured shape with rubberized grips, comfortable for long sessions. - Designed to suit both left- and right-handed users. - Smooth, accurate optical tracking. - Standard 3-button layout with scroll wheel. - Battery life up to 18 months. Wireless Receiver - One small USB receiver connects both devices. - Secure, long-range wireless operation (up to ~10 m). Compatibility - Works with Windows 7, Windows 8/8.1, Windows 10+, and ChromeOS. Durability - Spill-resistant keyboard surface. - Long-life keys suitable for heavy daily use. | 21 Pcs |
| 4 | Interactive Board | Display - Size: 86 inches diagonal - Resolution: 4K UHD (3840 × 2160) - Brightness: 400 cd/m² - Contrast Ratio: 5000:1 (dynamic) - Panel Type: TFT with Direct LED backlight - Viewing Angle: 178° horizontal and vertical - Aspect Ratio: 16:9 - Response Time: 6 ms - Display Colors: 10-bit (1.07 billion colors) - Glass: 4 mm tempered glass with zero bonding (MOHS hardness level 7) - Backlight Life: 50,000 hours Touch Capabilities - Technology: Infrared - Touch Points: Up to 40-point touch (40-point writing under Windows, 20-point under Android) - Touch Resolution: 32,768 × 32,768 pixels - Response Rate: <4 ms (single touch), <6 ms (multi-touch) - Accuracy: ≤1 mm - Recognition: Finger or passive stylus - Minimum Touch Object: 2 mm - Writing Height: 3 mm Connectivity & Compatibility - Supported Operating Systems: Windows 7/8/10, macOS, Android, Chrome OS, Linux - Interface: HID support for plug-and-play functionality - Power Supply: 100–240V AC, 50–60 Hz | 5 Pcs |
| 5 | Whiteboard Screen + Tripod | Device Type Portable projection screen, must be Tripod projection screen with adjustable stand Model Identification Must be Tripod Series portable projection screen (Elite Screens class) Screen Type Must be front projection portable screen with tripod stand Screen Sizes Must support multiple sizes from 50″ to 100″ diagonal Aspect Ratio Must support 16:9 aspect ratios Screen Material Must be matte white projection surface (MaxWhite or equivalent) Gain Must be 1.0–1.1 gain Resolution Support Must support Full projection compatibility Black Backing Must include black backing layer to prevent light penetration Border Must include 4-side black masking borders Stand Type Must include adjustable tripod stand with locking mechanism Height Adjustment Must support adjustable height with locking ring system Housing Must be metal casing with durable construction Portability Must be foldable and portable design for transport Setup Must support quick setup and dismantling Usage Environment Must support indoor and outdoor use Storage Must support compact storage after folding Accessories May include carrying bag / protective case Usage Must be suitable for meeting rooms, training, classroom, events, presentations, mobile setup Requirement Must be professional-grade tripod projection screen for frequent deployment | 1 Pcs |
| 6 | Laptop Basic | Processor: CPU must be Intel Core Ultra 7 / Core 7-240H / Ultra-series Intel processor class CPU Architecture: Processor shall be Intel hybrid architecture with AI / NPU acceleration support Chipset Platform: must be Intel SoC integrated chipset Graphics: Integrated graphics must be Intel Graphics / Intel Arc integrated GPU class RAM: Installed memory shall be 16 GB DDR5 5600 MHz minimum Memory Slots: Must support DDR5 SODIMM expansion up to ≥ 64 GB Storage: Internal storage shall be 512 GB PCIe Gen4 NVMe SSD minimum Storage Interface: Storage must be M.2 PCIe 4.0 NVMe Additional Storage: Platform must support dual M.2 NVMe slots Display Size: Display must be exactly 14.0-inch Display Resolution: Resolution must be 1920 × 1200 (WUXGA) Display Ratio: Aspect ratio shall be 16:10 Display Panel: Panel must be IPS anti-glare class Refresh Rate: Display must support 60 Hz minimum Wireless: Must support Wi-Fi 6 / Wi-Fi 6E (802.11ax) Bluetooth: Must support Bluetooth 5.2 or later Ethernet: Must include RJ-45 Gigabit LAN port 20 USB Ports: Must include USB-A + USB-C + HDMI + Audio combo USB-C Standard: USB-C must support Thunderbolt 4 / USB4 + DisplayPort + Power Delivery HDMI: Must support HDMI digital output up to 4K External Display Support: Must support displays via HDMI / USB-C / Thunderbolt Camera Integrated webcam: must be FHD 1080p with privacy shutter Audio: Stereo speakers with digital audio enhancement Keyboard: Backlit keyboard required Security: Must support TPM 2.0 + fingerprint reader + BIOS security features Power Adapter: Must include 65W USB-C power adapter with PD support Battery : Integrated battery must be ≥ 60 Wh class | 15 Pcs |
| 7 | External Monitor 24 ‘’ | Core Display - Panel Size: 24 inches (23.8″ viewable) - Resolution: 1920 × 1080 (Full HD) - Panel Type: IPS - Brightness: 220–300 nits typical range - Color Gamut: ~99% sRGB - Contrast Ratio: 1000:1 (IPS standard) - Viewing Angles: 178° / 178° - Refresh Rate: 60–120Hz depending on model - Response Time: 5ms class (GtG) Connectivity - HDMI: 1–2 ports - DisplayPort: DP 1.2/1.4 - USB Hub: Some models include multiple USB 3.0 downstream ports - Audio: Optional (line-out or built-in speakers) Ergonomics - Tilt: Adjustable ( −5° to +21°) - Height Adjustment: Available on productivity-grade models (up to ~130 mm) - Swivel: ±45° typical - Pivot: 90° rotation (portrait mode) on many IPS units - VESA: 100 × 100 mm supported Build & Features - Bezel: Slim 3-sided on modern IPS designs - Blue Light Reduction: Included (ComfortView / similar) - Flicker-Free: Yes (most IPS panels) - Stand: Stable productivity stand with cable management Typical Use-Cases - Office productivity - Dual-monitor setups - Light graphics work (99% sRGB IPS) - 1080p gaming up to 75–120Hz | 10 Pcs |
| 8 | Wireless Mousse | Tracking & Performance - Optical tracking sensor designed for smooth and consistent movement across most surfaces. - Standard office-grade sensitivity (around 1000 DPI), suitable for everyday productivity tasks. Wireless Connectivity - Fully wireless operation using a 2.4 GHz connection. - Includes a compact nano receiver that stays almost flush with the USB port. - Stable wireless range of up to 10 meters for unrestricted desk or meeting-room use. - Plug-and-play functionality — works instantly without driver installation. - Designed for long-distance reliability similar to higher-end productivity peripherals. Battery & Power - Powered by 1× AA battery, engineered for high efficiency. - Typical battery lifespan up to 12 months, depending on usage patterns. - Optimized sleep mode to conserve power during inactivity — common in premium productivity sets. Buttons & Controls - Standard 3-button configuration (Left / Right / Scroll-Click). - Smooth, easy-control scroll wheel ideal for documents and spreadsheets. Design & Comfort - Lightweight, compact design suitable for daily use at the office or on the go. - Ambidextrous shape for both left- and right-handed users. - Contoured sides designed for comfortable long-term use — stylistically similar to comfort-focused desktop combos. Compatibility Works seamlessly with Windows, macOS, ChromeOS, and Linux systems. | 35 Pcs |
| 9 | Smart TV 65 Inch | Display - Screen size: 65 inches - Smart TV operating system - High-definition LED panel Smart Features - Built-in Wi-Fi - Wireless display / screen mirroring support Connectivity - 2 × HDMI ports - 1 × USB port - Supports wireless casting Accessories & Requirements - 3-meter HDMI cable included - Wall-mount required (installation on wall) - Wall-mount bracket included Build & Usability Slim, wall-mount-ready design, Easy-to-navigate smart interface | 7 Pcs |
| 10 | Satellite Phones | Thuaray One (smartphone with satellilte compatibility) + Thuraya SIM Card | 3 Pcs |
| 11 | Laser printers | Core Functions - Multifunction Capabilities: Print, copy, scan, and fax - Print Technology: Color laser - Monthly Duty Cycle: Up to 20,000 pages (recommended monthly volume: 100–500 pages) - Processor Speed: 800 MHz Print & Copy Performance - Print Speed: Up to 18 pages per minute (black), 4 ppm (color) - First Page Out: As fast as 12.4 seconds (black) - Print Resolution: Up to 600 × 600 dpi - Copy Speed: Up to 14 cpm (black), 4 cpm (color) - Copy Features: Reduce/enlarge (25–400%), multi-copy up to 99 pages Paper Handling - Input Capacity: 150-sheet input tray - Output Capacity: 100-sheet output bin - Supported Media Sizes: A4, A5, A6, B5, envelopes - Supported Media Types: Plain, thick, thin, glossy, recycled, labels, envelopes Scan & Fax Features - Scanner Type: Flatbed and ADF (Automatic Document Feeder) - Scan Resolution: Up to 600 × 600 dpi - ADF Capacity: 40 sheets - Fax Speed: Up to 33.6 kbps - Fax Memory: Up to 400 pages Connectivity & Compatibility - Wireless: Wi-Fi 802.11 b/g/n - Wired: USB 2.0 and Ethernet LAN port - Mobile Printing Support: Apple AirPrint, Google Cloud Print, Mopria, and proprietary mobile app - Operating System Support: Windows, macOS, Linux Design & Power - Dimensions: Approx. 406 × 363 × 344 mm - Weight: Around 14 kg - Power Consumption: 300W (printing), 38W (ready), 1W (sleep) - Display: 2-line LCD with keypad navigation | 17 Pcs |
| 12 | Video conferencing Systems | Device Type: Video conferencing system; must be exactly GROUP Video Conferencing System System Type: Must be USB plug‑and‑play conference camera + speakerphone + hub system Video Resolution: Must support Full HD 1920 × 1080 @ 30 fps Camera Zoom: Must support 10× lossless HD zoom Field of View: Must be 90° field of view Pan / Tilt: Must support motorized pan 260° and tilt 130° Autofocus: Must include automatic focus camera system Video Codec: Must support H.264 UVC 1.5 with scalable video coding (SVC) Camera Presets: Must support 5 programmable camera presets Microphones: Must include 4 × omnidirectional microphones with ≥ 6 m (20 ft) pickup range Audio Type: Must support full‑duplex audio with acoustic echo cancellation and noise reduction Speakerphone: Must include integrated speakerphone with ultra‑wideband audio Audio Frequency: Must support 100 Hz – 11 kHz (microphone) and 120 Hz – 14 kHz (speaker) Connectivity: Must support USB 2.0 interface for PC / Mac connection Wireless Connectivity: Must support Bluetooth and NFC pairing Hub: Must include central hub for camera and speakerphone connectivity Cable Length: Must include ≥ 5 m cables (camera & speakerphone) and ≥ 3 m USB cable to host Control: Must include remote control and speakerphone touch controls Display Interface: Must include LCD display for call status and system information Mounting: Must support wall mount, table mount, and tripod thread Compatibility: Must support UVC‑compliant applications (Teams, Zoom, Webex, etc.) Expansion: Must support optional expansion microphones extending audio coverage to ≥ 8.5 m Power Supply: Must include external AC power adapter Operating Mode: Must support mid‑to‑large conference room deployment (≥ 14 participants class) Security: Must include Kensington lock slot support Usage: Must be suitable for conference rooms, boardrooms, NOC collaboration, and remote meetings | 3 Pcs |
| 13 | Speakers | Device Type: Karaoke amplifier with integrated speaker Portable: Yes Audio & Power Total Amplifier Power Output: 380 W Sound Settings: Bass, Treble, Echo Sound Effects: Built‑in party light effects Radio: Built‑in FM radio Microphone System - Microphones Included: must include 2 wireless microphones - Microphone Type: Wireless - Microphone Inputs: 2 inputs - Microphone Input Type: 3.5 mm (on amplifier) - Microphone Volume Control: Adjustable on amplifier Connectivity & Inputs Bluetooth: Supported Inputs: AUX (3.5 mm), USB‑C Recording Function: Supported (sound recording) Remote Control: Included Display & Design - Display: Built‑in status display (no lyrics screen) - Lighting: Integrated light effects Color Options: Black or Brown Power & Portability - Power Source: Rechargeable internal battery - Playable Formats: FM radio - Portability: Lightweight, carryable speaker design Physical Dimensions & Weight - Weight: Approximately 2.8 kg - Height: Approximately 23.0 cm - Width: Approximately 12.8 cm - Depth: Approximately 32.0 cm General Description - Battery‑powered portable karaoke PA system - Integrated speaker and amplifier with onboard controls - Supplied with 2 wireless microphones out of the box - Supports Bluetooth, USB, AUX, FM radio, and recording - Lightweight design for easy transport and use Recommended Usage - Home karaoke - Kids events - Small gatherings and parties | 4 Pcs |
| 14 | Mobile Stand for interactive Board | Device Type: Mobile floor stand for active board / interactive flat panel Compatible Display Size: Must support interactive displays from 65‑inch to 86‑inch class Mounting Type: Floor‑standing mobile mount with display bracket Maximum Load Capacity: Must support ≥ 100 kg load bearing VESA / Hole Pattern: Must support 210 × 180 mm mounting pattern Material: Must be constructed from high‑quality carbon steel with tempered glass components Mobility: Must include heavy‑duty caster wheels with foot brake locking mechanism Height Adjustment: Must support adjustable mounting height for flexible viewing and usage Shelf: Must include integrated accessory shelf suitable for OPS / laptop / peripherals Product Dimensions: Approximately 1540 × 1080 × 580 mm (H × W × D) Package Dimensions: Approximately 1590 × 690 × 155 mm Net Weight: Approximately 33.7 kg ±1 kg Gross Weight: Approximately 37.9 kg ±1 kg Color: Black / Silver professional finish Design: European minimalist industrial design suitable for professional environments Installation: Must support quick installation and secure mounting of active boards Operating Environment: Must be suitable for classroom, meeting room, training room, and conference use Application: Must support educational, corporate, collaboration, and presentation environments | 2 Pcs |
| 15 | Laptop Standard Specs | Processor CPU must be Intel Core Ultra 7 / Core 7 H-Series platform - CPU Architecture Processor shall be hybrid multi-core architecture with performance + efficiency cores - AI / NPU Platform must support Intel AI Boost / NPU acceleration - Graphics Integrated graphics must be Intel Graphics / Intel Arc integrated GPU class - RAM Installed memory shall be 16 GB DDR5 5600 MHz minimum - Memory Slots Must support DDR5 SODIMM expansion up to ≥ 64 GB - Storage Internal storage shall be 512 GB PCIe Gen4 NVMe SSD minimum - Storage Interface Storage must be M.2 PCIe 4.0 NVMe - Additional Storage Slot Platform must support dual M.2 NVMe slots - Display Size Display must be exactly 16.0-inch - Display Resolution Resolution must be 1920 × 1200 (WUXGA) minimum - Display Ratio Aspect ratio shall be 16:10 - Display Panel Panel must be IPS anti-glare class - Refresh Rate Display must support 60 Hz minimum - Wireless Must support Wi-Fi 6 / Wi-Fi 6E / Wi-Fi 7 class - Bluetooth Must support Bluetooth 5.3 or later - Ethernet Must include RJ-45 LAN port integrated - USB Ports Must include USB-A + USB-C + HDMI + Audio combo - USB-C Standard USB-C must support Power Delivery + DisplayPort + high-speed data - Thunderbolt Must support Thunderbolt 4 or USB4 class port - HDMI Must support HDMI digital output - Camera Integrated webcam must be FHD 1080p with privacy shutter - Audio Stereo speakers with Dolby / digital audio support - Keyboard Full keyboard with numeric keypad required - Security Must support TPM 2.0 + fingerprint reader support - Battery Integrated battery must be ≥ 64 Wh class - Power Adapter Must include USB-C or slim adapter with fast charging support - External Display Support Must support multiple external displays via HDMI / USB-C / Thunderbolt | 5 Pcs |
| 16 | Laptop Medium Specs | - Processor: CPU must be Intel Core Ultra 7-155U / 165H class processor - CPU Architecture: Processor shall be Intel Core Ultra hybrid architecture with NPU / AI engine - Chipset: Platform must be Intel integrated SoC platform - Graphics: Integrated graphics must be Intel Graphics / Intel Arc class GPU - RAM: Installed memory shall be 16 GB DDR5 5600 MHz minimum - Memory Slots: Must support DDR5 SODIMM expansion up to ≥ 64 GB - Storage: Internal storage shall be 512 GB PCIe Gen4 NVMe SSD minimum - Storage Interface: Storage must be M.2 PCIe 4.0 NVMe - Display Size: Display must be exactly 16.0-inch - Display Resolution: Resolution must be 1920 × 1200 (WUXGA) minimum - Display Ratio: Aspect ratio shall be 16:10 - Display Panel: Panel must be IPS anti-glare, ≥ 300 nits class - Optional High-Resolution: Platform shall support 2560 × 1600 or higher panel option - Refresh Rate: Display must support 60 Hz minimum - Wireless: Must support Wi-Fi 6E (802.11ax) - Bluetooth: Must support Bluetooth 5.3 or later - Ethernet: Must include RJ-45 LAN port integrated - USB Ports: Must include USB-A + USB-C + HDMI + Audio combo - USB-C Standard: USB-C must support Thunderbolt 4 / USB4 + DisplayPort + Power Delivery - HDMI: Must support HDMI digital output - Camera: Integrated webcam must be FHD 1080p with privacy shutter - Audio: Stereo speakers with digital audio enhancement - Keyboard: Full keyboard with numeric keypad required - Security: Must support TPM 2.0 + fingerprint reader + IR camera support - Operating System: Must be delivered with Windows 11 Pro 64-bit - Battery: Integrated battery must be ≥ 86 Wh class - Power Adapter: Must include USB-C fast charging adapter External Display Support: Must support multiple displays via HDMI / USB-C / Thunderbolt | 8 Pcs |
| 17 | Tablets Standard | - Network: Must support 5G / LTE / 3G / 2G mobile network with Nano-SIM - Processor: CPU must be MediaTek MT8775 / Dimensity 7300 class octa-core processor - CPU Architecture: Processor shall be octa-core high-performance architecture (A78 + A55 class) - RAM: Installed memory shall be 6 GB minimum - Storage: Internal storage shall be 128 GB flash storage - External Storage: Must support microSD expansion ≥ 1 TB - Display Size: Display must be exactly 11.0-inch TFT LCD - Display Resolution: Resolution must be 1920 × 1200 (WUXGA) - Refresh Rate: Display must support 90 Hz - Aspect Ratio: Screen ratio shall be 16:10 - Rear Camera: Rear camera must be 8 MP autofocus - Front Camera: Front camera must be 5 MP - Battery: Battery capacity must be 7040 mAh minimum - Charging: Must support USB-C fast charging up to 25W - Wireless: Must support Wi-Fi 5 / Wi-Fi 6 class - Bluetooth: Must support Bluetooth 5.3 or later - USB Port: Must include USB Type-C port - Audio: Must include dual speakers with Dolby audio support - Sensors: Must include accelerometer, gyro, compass, proximity (cellular) - Operating System: Must be delivered with Android 16 / One UI 8 or later - Connectivity Ports: Must include SIM slot + microSD slot - Security: Must support face unlock / device security platform - Productivity Features: Must support multi-window, split screen, productivity mode - Accessories Support: Must support keyboard cover / protective cover ecosystem Adapter, cover, and screen protection included | 58 Pcs |
| 18 | Tablet High Specs | - Device Type: High-performance tablet device - Processor: CPU must be MediaTek Dimensity 9400+ flagship processor - CPU Architecture: Processor shall be octa-core 3 nm class high-performance architecture - AI / NPU: Platform must support integrated AI / NPU acceleration engine - RAM: Installed memory shall be 12 GB RAM minimum - Storage: Internal storage shall be 256 GB flash storage - External Storage: Must support microSD expansion - Display Size: Display must be exactly 14.6-inch Dynamic AMOLED 2X - Display Resolution: Resolution must be 2960 × 1848 pixels - Display Refresh Rate: Display must support 120 Hz adaptive refresh rate - Display Brightness: Display shall support High Brightness Mode ≥ 1000 nits class - Touch / Pen & Keyboard Support: Must support active stylus, S-Pen, with original keyboard included - Rear Camera: Dual rear camera system, 13 MP + 8 MP - Front Camera: Dual front camera configuration - Battery: Battery capacity must be ≈ 11,600 mAh class - Charging: Must support fast charging via USB-C - Wireless: Must support Wi‑Fi 6E / Wi‑Fi 7 class - Bluetooth: Must support Bluetooth 5.4 or later - USB Port: Must be USB‑C with high-speed data and charging - Audio: Quad speaker system with spatial audio - Protection: Device must support IP68 water and dust resistance - Security: Must support biometric authentication - Operating System: Must be delivered with Android 16 / One UI 8 or later - Productivity Features: Must support desktop mode, multi-window, and stylus input - Accessories Support: Must support keyboard cover and stylus ecosystem Adapter, cover, and screen protection included | 6 Pcs |
| 19 | External Monitor 27 ‘’ | - Device Type: Professional monitor display; must be exactly 27‑inch 4K UHD IPS USB‑C monitor class - Panel Size: Display must be exactly 27‑inch - Resolution: Native resolution must be 3840 × 2160 (4K UHD) - Refresh Rate: Refresh rate must be 60 Hz - Panel Type: Panel must be IPS technology - Color Gamut: Must support ≥ 99% sRGB and ≥ 95% DCI‑P3 - Brightness: Brightness must be ≥ 350 nits - Contrast Ratio: Contrast ratio must be 1000:1 minimum - Viewing Angle: Must support 178° horizontal / 178° vertical viewing angle - HDR Support: Must support HDR10 - Tilt: Must support −5° to +25° tilt adjustment - Swivel: Must support ±30° swivel - Pivot: Must support 360° pivot / portrait rotation - Height Adjustment: Must support ≥ 120 mm height adjustment - VESA Mount: Must support 100 × 100 mm VESA mounting - USB‑C Port: Must support USB‑C with DisplayPort Alt Mode, ≥ 65W Power Delivery, and USB data - HDMI Ports: Must include 2 × HDMI 2.0 supporting 4K @ 60 Hz - DisplayPort: Must include 1 × DisplayPort 1.4 supporting 4K @ 60 Hz - USB Hub: Must include ≥ 2 × USB‑A 3.2 ports - Audio Output: Must include 3.5 mm audio output - Speakers: Must include built‑in speakers ≥ 2 × 3W or optional integrated audio - Bezel: Must have 3‑side ultra‑thin bezel design - Stand: Must include metal stand with stability support - Eye Protection: Must support low blue light mode and flicker‑free technology - Cable Management: Must include integrated cable management channel Usage: Must be suitable for professional office, design, GIS, development, and multi‑monitor setup | 9 pcs |
| 20 | Portable monitor | Panel Size: Display must be exactly 15.6‑inch Resolution: Native resolution must be 1920 × 1080 (Full HD) Refresh Rate: Must be 60 Hz Panel Type: Must be IPS panel technology Brightness: Must be 250 nits Contrast Ratio: Must be 1000:1 Response Time: Must be 4 ms (GtG) Aspect Ratio: Must be 16:9 Viewing Angle: Must support 178° horizontal / 178° vertical Surface Treatment: Must include anti‑glare coating Color Depth: Must support 16.7 million colors, 8‑bit (6‑bit + FRC) HDMI Port: Must include 1 × HDMI 2.0b (Full HD @ 60 Hz) USB‑C Ports: Must include 2 × USB Type‑C with DisplayPort Alt Mode and up to 15W power delivery / input Audio: Must include 2 × built‑in speakers (1 W each) Power Input: Must be powered via USB‑C external power (no internal battery) Mounting: Must support 75 × 75 mm VESA mount Tripod Support: Must include standard 1/4‑inch tripod socket Tilt Adjustment: Must support 0° to 90° adjustable tilt stand Connectivity Mode: Must support single USB‑C cable for power and display (DisplayPort Alt Mode) Accessories: Must include USB‑C cable, HDMI cable, protective pouch, and quick start guide Operating Mode: Must support plug‑and‑play operation without driver installation Sync Technology: Must support Adaptive‑Sync (48–60 Hz range) Usage: Must be suitable for mobile workstation, IT field work, NOC, presentations, and laptop screen extension | 10 Pcs |
| 21 | Mobile Phone | Network Technology GSM / HSPA / LTE / 5G Body Dimensions 161.1 x 77.4 x 8.2 mm (6.34 x 3.05 x 0.32 in) Weight 213 g (7.51 oz) Build Glass front (Gorilla Glass Victus+), glass back (Gorilla Glass), aluminum frame SIM Nano-SIM + Nano-SIM + eSIM (max 2 at a time) IP67 dust/water resistant (up to 1m for 30 min) Display Type Super AMOLED, 120Hz, HDR10+, 1000 nits (HBM) Size 6.6 inches, 106.9 cm2 (~85.8% screen-to-body ratio) Resolution 1080 x 2340 pixels, 19.5:9 ratio (~390 ppi density) Protection Corning Gorilla Glass Victus+ Platform OS Android 14, up to 4 major Android upgrades, One UI 6.1 Chipset Exynos 1480 (4 nm) CPU Octa-core (4x2.75 GHz Cortex-A78 & 4x2.0 GHz Cortex-A55) GPU Xclipse 530 Memory Card slot microSDXC (uses shared SIM slot) Internal 256GB 12GB RAM Main Camera Triple 50 MP, f/1.8, (wide), 1/1.56", 1.0µm, PDAF, OIS Features LED flash, panorama, HDR Video 4K@30fps, 1080p@30/60fps, gyro-EIS Sound Loudspeaker with stereo speakers Comms WLAN Wi-Fi 802.11 a/b/g/n/ac/6, dual-band, Wi-Fi Direct Bluetooth 5.3, A2DP, LE Positioning GPS, GALILEO, GLONASS, BDS, QZSS NFC Yes (market/region dependent) USB USB Type-C 2.0, OTG Features Sensors Fingerprint (under display, optical), accelerometer, gyro, compass, Virtual proximity sensing,Circle to Search Battery Type Li-Ion 5000 mAh Charging 25W wired Battery Active use score 13:27h The supplier must make sure that their price covers the Mobile, Adapter, cover, and screen protection | 22 Pcs |
| 22 | Wireless access Points + power bank | 4G LTE Router - Device Type: 4G LTE Router - Model Identification - Network Type: Must support 4G LTE Cat4 mobile network with SIM card slot - LTE Speed: Must support LTE download speeds up to 150 Mbps - Wi-Fi Standard: Must support IEEE 802.11 b/g/n - Wi-Fi Frequency: Wireless band shall be 2.4 GHz only - Wireless Speed: Wireless speed must be up to 300 Mbps (802.11n) - LAN Ports: Must include 3 × 10/100 Mbps LAN ports - WAN/LAN Port: Must include 1 × 10/100 Mbps LAN/WAN port - SIM Slot: Must include 1 × Nano / Micro SIM slot for LTE connection - Antennas: Must include 2 detachable external 4G LTE antennas - External Antenna Interface: Must support SMA antenna connectors - Security: Must support WEP, WPA, WPA2, WPA-PSK, and WPA2-PSK encryption - Connected Devices: Must support a minimum of 32 wireless clients simultaneously - Buttons: Must include WPS, Reset, and Wi-Fi ON/OFF buttons - Power Supply: Must operate using 9V DC external adapter - Router Mode: Must support 4G router mode and wired WAN router mode - DHCP / NAT: Must support DHCP server, NAT, and routing functions - Firewall: Must support SPI firewall, access control, and filtering - Management: Must support web-based configuration interface - Operating Mode: Must support plug-and-play SIM auto-configuration - Certification: Must support LTE FDD / TDD compatibility for global networks Mini DC UPS Power Backup Unit - Device Type: Mini DC UPS power backup unit - Model Identification - Battery Capacity: Internal battery must be minimum 8,800 mAh lithium - Input Voltage: Input power shall support AC 100–240V, 50/60Hz adapter input - Output Interface: Must include USB, DC, and POE output ports - USB Output: Must provide 5V DC USB output - DC Output: Must support selectable 9V / 12V DC output - POE Output: Must support 15V POE and 24V POE output via RJ45 - POE Data Speed: POE port must support 10/100 Mbps data transmission - Output Current: Must support - 5V ≥ 1A - 9V ≥ 1A - 12V ≥ 1A - 15V ≥ 0.6A - 24V ≥ 0.6A - Maximum Power: Maximum output power shall be approximately 17W class - Protection: Must include over-voltage, overload, short-circuit, and over-charge protection - Operating Mode: Must support simultaneous charging and power supply (true UPS mode) - Application: Must support powering router, modem, IP camera, ONU, switch, and wireless devices - Indicators: Must include LED status indicators for power and battery - Backup Function: Must provide automatic switchover to battery during power failure - Output Stability: Must provide regulated DC output without interruption - Connectivity Type: Must support POE power and data through a single Ethernet cable - Safety: Must include battery protection circuit and automatic shutdown protection - Charging Type: Must support external DC adapter charging with internal battery management | 4 Pcs |
| 23 | LTE/4G/5G Routers | Device Type Portable 4G LTE mobile Wi-Fi router Model Identification Network Type Must support 4G LTE / 3G / 2G mobile network with SIM card slot LTE Speed Must support LTE download up to 150 Mbps and upload up to 50 Mbps Wi-Fi Standard Must support IEEE 802.11 b/g/n (2.4 GHz) Wireless Speed Wireless speed must be up to 300 Mbps class Connected Devices Must support minimum 10 wireless devices simultaneously Frequency Band Must support 2.4 GHz wireless band Display Must include integrated screen display for status / signal / battery / data usage Battery Must include 2100 mAh rechargeable battery Battery Runtime Must support up to 10 hours operating time Charging Interface Must support USB charging input Storage Slot Must include microSD slot supporting up to 2 TB storage Antenna Must include internal antennas Management Must support web interface and mobile app management Security Must support WPA / WPA2 wireless security SIM Type Must support Nano-SIM / Micro-SIM (adapter supported) Operating Mode Must support portable hotspot / travel router mode Power Source Must operate on battery or USB power Usage Must be suitable for field work, emergency connectivity, travel, NOC backup, mobile internet Requirement Must allow plug-and-play SIM operation without external modem | 27 Pcs |
| 24 | Satellite Internet Terminals (StarLink) | Satellite Internet Terminals (StarLink) | 4 Pcs |
| 25 | IP Phone | IP Desk Phone (PBX-Compatible) - Device Type: IP desk phone, must be compatible with the IP PBX - Model Identification - SIP Accounts: Must support 6 SIP accounts, 3 lines, and up to 3 call appearances - Display: Must include 2.7-inch graphical LCD display (132 × 64) - Audio: Must support HD audio on handset and speakerphone - Speakerphone: Must support full-duplex speakerphone - Conferencing: Must support 5-way audio conferencing - Network Ports: Must include 2 × 10/100/1000 Mbps Ethernet ports - PoE: Must support Power over Ethernet (PoE) - Headset Support: Must include RJ9 headset port with EHS support - Provisioning: Must support zero-touch provisioning and cloud management - Device Management: Must support centralized management via device management system (GDMS) - Security: Must support secure boot, dual firmware image, encrypted data storage, TLS, and SRTP - Noise Reduction: Must support noise shield / background noise reduction technology - Programmable Keys: Must include XML programmable soft keys - Phonebook / Call Logs: Must support call history, phonebook, and speed dial - Mounting: Must support desk stand and wall mounting - Power Input: Must support PoE or external DC power adapter - Firmware Upgrade: Must support remote firmware upgrade - Protocols: Must support SIP, RTP, TCP/IP, UDP, HTTP, HTTPS, DHCP, DNS, SRTP, and TLS Usage: Must be suitable for IP PBX, VoIP system, call center, and office telephony | 15 Pcs |
| 26 | GrandStream PBX | IP PBX Unified Communication Appliance - Device Type: IP PBX unified communication appliance - Model Identification - System Type: Must be on-premise IP PBX supporting voice, video, data, and collaboration services - Maximum Users: Must support ≥ 2000 SIP users / extensions - Concurrent Calls: Must support ≥ 300 concurrent calls - SIP Trunks: Must support ≥ 200 SIP trunk accounts - FXO Ports: Must include 4 × RJ11 FXO ports for PSTN lines - FXS Ports: Must include 4 × RJ11 FXS ports for analog phones - Network Ports: Must include 3 × Gigabit RJ45 Ethernet ports - PoE Support: Must support PoE+ on network interface - USB Ports: Must include 2 × USB 3.0 ports - Storage: Must support SD card storage expansion - Conferencing: Must support built-in audio and video conferencing platform - Video Conference: Must support multiple video rooms and HD conferencing - Codec Support: Must support Opus, H.264, H.265, H.263, and VP8 codecs - Security: Must support TLS, SRTP, HTTPS, secure boot, and certificate authentication - Remote Access: Must support NAT traversal, remote connect, and cloud management - Management: Must support web interface, API, and cloud management platform - Provisioning: Must support auto-provisioning for SIP endpoints - Call Features: Must support IVR, call queue, call recording, CDR, and voicemail - Integration: Must support CRM, PMS, and third-party API integration - Display: Must include front LCD display with status indicators - Mounting: Must support rack-mount installation - Power Input: Must operate using 12V DC external adapter - Operating System: Must be based on an Asterisk-based telephony platform - Application: Must be suitable for enterprise telephony, call center, unified communication, and VoIP systems | 1 Pc |
| 27 | Shredder | Core Specs - Security Level: P‑5 (particle cut) - Cut Type / Size: 2 × 15 mm particle cut - Max Sheet Capacity (Auto‑Feed): Up to 300 sheets (80 GSM) - Manual Feed Capacity: 9–11 sheets (80 GSM) - Waste Bin Volume: 40 L - Noise Level: 60 dB - Shreds: Paper, paper clips, credit cards - Operation Time (Auto feed): 480 minutes continuous - Power: 230 V AC Dimensions: Width: 425 mm * Height: 696 mm * Depth: 351 m Key Features - Auto feed mechanism (lockable tray, 300-sheet stack) - MHP cutting rollers (High‑Performance steel rollers) - Oil‑free operation (maintenance‑free) - Jam‑resistant system with automatic reverse - Auto start/stop with photoelectric sensor - Overheat protection - Bin‑full safety shut‑off - Pull‑out waste container - Quiet operation | 2 Pcs |
| 28 | HDMI Cable 1.5 m | Length - 1.5/3/5 meters Connector Type - HDMI to HDMI (Standard Type-A) Version - HDMI 2.0 or higher (supports 4K@60Hz) (Most 1.5m cables today meet this by default) Supported Features - 4K video output - Audio + video in one cable - HDR compatibility (depends on device support) - ARC/eARC support (cable-dependent but typically present in newer cables) - 3D video support Build - Gold-plated connectors (common standard) - Shielded cable for reduced interference - Durable molded ends Use Case - Connecting laptops, PCs, TVs, monitors, projectors, docks, consoles | 7 Pcs |
| 29 | HDMI Cable 3 m |  | 7 Pcs |
| 30 | HDMI Cable 5 m |  | 6 Pcs |
| 31 | Type C to 4 HDMI Splitter | - Device Type: USB‑C multi‑display adapter (USB‑C to 4‑Port HDMI adapter) - Model Identification - Host Interface: Must use USB‑C 3.1 / USB‑C Gen1 male connector - Video Outputs: Must include 4 × HDMI female ports - Maximum Displays: Must support 4 external displays simultaneously - Maximum Resolution: Must support 1920 × 1080 @ 60 Hz per port - Refresh Rate: Must support 60 Hz maximum refresh rate - Audio Support: Must support embedded audio via HDMI (2.1 channel) - Power Source: Must be USB bus‑powered (no external adapter required) - Power Consumption: Must not exceed approximately 900 mA from USB‑C port - Display Modes: Must support mirror, extend, primary, and rotation modes - Driver Support: Must support Windows and macOS driver installation - Software Support: Must support multi‑monitor management utility - Housing: Must be aluminum enclosure for durability - Cable: Must include integrated USB‑C cable ≥ 50 cm - Operating Temperature: Must support 0 °C to 45 °C operating range - Storage Temperature: Must support −10 °C to 71 °C storage range - Compatibility: Must support laptop and desktop systems with USB‑C display output - Mounting / Usage: Must support desktop workstation multi‑monitor setup - Usage: Must be suitable for control room, GIS, monitoring, trading, NOC, development, and multi‑screen workstation environments Warranty: Must include minimum 2‑year warranty | 5 Pcs |
| 32 | Portable Projector | • Device Type: Smart portable projector • Model Identification • Projection Technology: Must be DLP projection system with 0.23″ DMD chip • Native Resolution: Must be 1920 × 1080 Full HD • Brightness: Brightness must be 400 ANSI lumens • Light Source: Must be RGB LED light engine • Light Source Life: Light source lifetime must be up to 30,000 hours • Contrast Ratio: Contrast ratio must be 400:1 class • Aspect Ratio: Native aspect ratio shall be 16:9 • Throw Ratio: Throw ratio must be 1.2:1 • Projection Size: Must support projection size from 30″ to 150″ • Auto Adjustment: Must support auto focus, auto keystone, auto screen fit, and obstacle avoidance • Operating System: Must include Android TV / Google TV built in system • CPU: Processor must be MT9629 platform • Memory: Must include 2 GB RAM and 16 GB internal storage • Wireless: Must support Wi Fi screen casting / Chromecast built in • Bluetooth: Must support Bluetooth 5.1 or later • HDMI: Must include HDMI input • USB: Must include USB port for media playback and power • Audio: Must include dual speakers 8W × 2 with Dolby audio support • Battery: Must include built in battery supporting portable operation • Playback Time: Battery must support approximately 2.5 hours of video playback • Smart Features: Must support automatic environment adaptation setup • Connectivity: Must support HDMI, USB, Wi Fi, Bluetooth, and wireless casting • Operating Mode: Must support stand alone projection without external PC | 6 Pcs |
| 33 | External hard drives | Capacity - Storage Size: 2 TB - Usable Space: Approx. 1.8–1.9 TB (after formatting) Performance - Read Speed: ~1,000–2,000 MB/s (depending on interface) - Write Speed: ~900–2,000 MB/s - Drive Type: NVMe or SATA - Interface: USB 3.2 Gen 2 (10 Gbps) or USB‑C (Type‑C) Physical - Form Factor: Portable external SSD - Dimensions: Typically compact (approx. 50–100 mm length) - Weight: ~30–80 g - Casing: Aluminum or rugged polymer - Shock Resistance: Drop‑resistant up to 2 meters (typical) Connectivity - Connector Type: USB‑C - Cable Support: USB‑C to USB‑C / USB‑C to USB‑A - Backward Compatibility: USB 3.0 and USB 2.0 (reduced speeds) Security - Optional: AES 256‑bit hardware encryption - Password protection: Supported on many models Power - Power Source: USB‑powered (no external adapter) - Power Requirements: 5V via USB port Environmental & Durability - Operating Temperature: Usually, 0°C to 45°C - Storage Temperature: –20°C to 70°C - Resistance: - Water resistance - Dust resistance - Shock protection - Vibration protection | 10 Pcs |
| 34 | GIS Workstations | Processor (CPU) 12-core / 24-thread desktop-grade processor with 3D V-Cache; clock speed up to ~5.5GHz Motherboard ATX form factor supporting DDR5 memory, PCIe 5.0, NVMe SSDs, and compatible with modern AMD processors Memory (RAM) 64GB DDR5 (2×32GB), dual-channel, ~6400MT/s nvidia 5080Graphics Card High-end GPU with ray tracing and AI acceleration; next-gen archi"Processor (CPU) 12-core / 24-thread desktop-grade processor with 3D V-Cache; clock speed up to ~5.5GHz Motherboard ATX form factor supporting DDR5 memory, PCIe 5.0, NVMe SSDs, and compatible with modern AMD processors Memory (RAM) 128GB DDR5 (2×64GB), dual-channel, ~6400MT/s nvidia 5080Graphics Card High-end GPU with ray tracing and AI acceleration; next-gen architecture, 16GB–24GB GDDR6X memory Primary Storage Kingston 2TB PCIe Gen 4 NVMe SSD — optimized for fast boot and read/write Secondary Storage 2TB SATA SSD — ideal for bulk storage Cooling System 360mm all-in-one liquid cooling with radiator and three fans Power Supply Unit 1200W, 80+ Gold certified, modular, supports GPU with high power draw Case Mid or full tower, supports liquid cooling, includes 4 pre-installed fans for airflow Wireless Onboard Wi-Fi 6/6E and Bluetooth 5.x support Network Expansion PCIe-based 10GbE network card alongside integrated LAN Performance & Capabilities • Ultra-high performance in gaming, rendering, and streaming • Supports multi-monitor setups at 4K or higher • Excellent thermals with water cooling and airflow design • Future-proof with PCIe 5.0 and DDR5 support • Ideal for productivity, VR, AI, and intensive workloads" tecture, 16GB–24GB GDDR6X memory Primary Storage 2TB PCIe Gen 4 NVMe SSD — optimized for fast boot and read/write Secondary Storage 2TB SATA SSD — ideal for bulk storage Cooling System 360mm all-in-one liquid cooling with radiator and three fans Power Supply Unit 1200W, 80+ Gold certified, modular, supports GPU with high power draw Case Mid or full tower, supports liquid cooling, includes 4 pre-installed fans for airflow Wireless Onboard Wi-Fi 6/6E and Bluetooth 5.x support Network Expansion PCIe-based 10GbE network card alongside integrated LAN Performance & Capabilities • Ultra-high performance in gaming, rendering, and streaming • Supports multi-monitor setups at 4K or higher • Excellent thermals with water cooling and airflow design • Future-proof with PCIe 5.0 and DDR5 support • Ideal for productivity, VR, AI, and intensive workloads | 3 Pcs |
| 35 | Docking Station | Design & Build - Compact, lightweight aluminum housing with passive cooling design - Plug-and-play functionality, no driver installation required - LED indicator for power and connectivity status Ports & Connectivity - Video Outputs: 1× HDMI 2.0 (up to 4K @ 60Hz), 1× DisplayPort 1.2 (up to 4K @ 60Hz) - USB Ports: 2× USB-A 3.2 Gen 1, 1× USB-C (data + power delivery) - Ethernet: RJ45 Gigabit Ethernet port for wired networking - Audio: 3.5mm combo audio jack (headset/microphone support) - SD/MicroSD: Dual card reader slots supporting UHS-I standard Power & Charging - Supports USB-C Power Delivery up to 100W (laptop charging + peripheral power) - Intelligent power distribution to connected devices Compatibility - Works with Windows, macOS, ChromeOS, and Linux systems - Backward compatible with USB 2.0/3.0 devices - Supports multi-monitor setups (mirror or extend mode) Security & Features - Hot-swap capability for USB devices - Overcurrent and surge protection Universal plug-and-play recognition | 2 Pcs |
| 36 | Webcam | Device Type USB conference webcam Model Identification Image Resolution: Must support 1920 × 1080 Full HD video Frame Rate: Must support 1080p @ 30 fps and 720p @ 60 fps Sensor Must include 4 MP image sensor Field of View Must support 90° field of view (adjustable 65° / 78° / 90°) Focus Must support autofocus with glass lens HDR / Light Correction: Must support automatic light correction (Right Light class) with HDR support Auto Framing: Must support auto framing / face tracking Microphones: Must include dual noise-reducing microphones Privacy Must include integrated privacy shutter Interface Must use USB-C connection with plug-and-play support Cable: Must include integrated USB-C cable with USB-A adapter support Mounting: Must support monitor clip + tripod mount Compatibility : Must support Windows / macOS / ChromeOS / Linux / Teams / Zoom / Meet Audio: Must support built-in microphone with echo and noise reduction Software Support:Must support camera control software / firmware update utility Power: Must be powered via USB bus power Operating Mode: Must support plug-and-play video conferencing without driver installation Usage: Must be suitable for video conferencing, streaming, office, call center, and remote meetings | 2 Pcs |
| 37 | Portable Printer with battery | Print Quality & Technology Print Resolution: Up to 4800 × 1200 dpi Ink System: Dual-cartridge design, - Black (Pigment BK) - Color (BK, C, M, Y) High-precision inkjet technology optimized for documents and photos Print Performance • Mono Print Speed: approx. 9.0 ipm • Color Print Speed: approx. 5.5 ipm • Photo Printing: 10×15 cm borderless print in < 60 seconds Paper Handling • Borderless Printing Sizes: A4, Letter, 4"x6", 5"x7", 7"x10", 8"x10", 5"x5", 3.5"x3.5" • Two-Sided Printing: Manual duplex support • Paper Input Capacity: Rear tray – up to 50 sheets (plain paper) Supported Paper Sizes: – A4, A5, B5, Letter, Legal – Envelopes: DL, COM10 – Square formats: 5"x5", 3.5"x3.5" – Card size: 91 × 55 mm – Photo sizes: 4"x6", 5"x7", 7"x10", 8"x10" Display & Controls • 1.44" (3.6 cm) OLED Monochrome Display • Intuitive interface for mobile and standalone printing Connectivity • USB-C (Hi-Speed USB) • Wi-Fi (IEEE 802.11 a/b/g/n) • Wi-Fi Security: WPA-PSK, WPA2-PSK, WEP, Admin password • Wireless Bands: 2.4 GHz + 5 GHz (EU GB/EUR models) Operating System Compatibility • Windows: Windows 10, Windows 8.1, Windows 7 SP1 • macOS: macOS 10.11.6 to macOS 10.15 • Mobile Devices: iOS, Android, Windows 10 Mobile, Fire OS Battery & Power • Battery Type: Lithium-ion • Battery Rating: DC 10.8V, 2170mAh • Charging Options: USB charging + AC adapter • Charging Time: – < 2 hours 30 min (AC adapter) – < 6 hours (USB 1.5A) • Battery Indicator: 4-step LED system • Printable Yield: approx. 330 pages (via USB connection to PC) Physical Characteristics • Portable, lightweight design • Total weight: < 2.5 kg • Portable with built in battery | 2 Pcs |
| 38 | Laptop Backpack | Core Features - Weather‑resistant exterior engineered to keep devices dry - Dedicated padded compartment fits up to 16-inch laptops - Front utility pocket for quick‑access essentials - Lightweight construction for daily commuting, office, travel, or hybrid‑work environments Design & Build - Dimensions (W × D × H): 310 × 120 × 445 mm - Dimensions in inches: 12.2" × 4.7" × 17.5" - Weight: Starting at 440 g (0.97 lbs) - Exterior Material: 80% RPET + 20% Polyester - Lining Material: 100% Polyester - Color: Black Functionality - Spacious main compartment for daily essentials - Dedicated padded laptop sleeve - Additional front pocket for accessories - Durable stitching and reinforced structure - Lightweight and comfortable for extended wear Designed with sustainability and long‑term usability in mind | 8 Pcs |
| 39 | MousePad | Size: Large / Extended (minimum 800 × 300 mm) Surface: Smooth fabric surface optimized for optical and laser mice Base: Non-slip rubber base Edge: Stitched edges for durability | 4 Pcs |
| 40 | Ultra wide monitor 34 inch | Core Display - Panel Size: 34 inches (UltraWide) - Resolution: 2560 × 1080 (WFHD, 21:9) - Panel Type: IPS - Brightness: 400 nits - Color Gamut: 99% sRGB - Contrast Ratio: 1000:1 - Viewing Angles: 178° / 178° - Refresh Rate: 100Hz - HDR Support: Basic HDR / HDR400 class - Surface: Anti-glare Connectivity - USB-C: Video + data (single-cable connection) - HDMI: Dual HDMI inputs - DisplayPort: One DisplayPort input - Audio: Built-in stereo speakers Ergonomics - Tilt Adjustment: Yes - Height Adjustment: Yes - Stand Type: Stable, low-wobble design - VESA Mounting: 100 × 100 mm Features - Adaptive Sync: Compatible with FreeSync-type technologies - On-Screen Control: Layout splitting & quick configuration - Blue Light Reduction Mode: Yes - Flicker-Free Backlight: Yes Physical Design - Color: White - Bezel: Slim 3-sided - Chassis: Matte finish | 4 Pcs |
| 41 | Power bank 250 W | Core Power • Battery Capacity: 27,650mAh (3,950mAh × 7 cells), 99.54Wh TSA-approved for air travel • Total Power Output: 250W max multi-device fast charging • Designed for laptops, tablets, phones, and high-demand electronics Ports & Output Performance • 3 Total Output Ports: – USB-C1: Up to 140W (PD 3.1) – USB-C2: Up to 140W – USB-A: Up to 22.5W high-speed charging • Voltage/Current Support: – 5V/3A ¦ 9V/3A ¦ 12V/3A ¦ 15V/3A ¦ 20V/5A ¦ 28V/5A – USB-A supports up to 6A depending on protocol Recharge (Input) • 170W USB-C fast recharge capability • Full recharge time: ≈ 37–43 minutes with high-power dual-USB-C input Smart Features • Integrated Bluetooth Smart App – Real-time power statistics – Device charging optimization – Power bank locator with sound alert – Smart charging to extend battery health of connected devices • ActiveShield 2.0 thermal and safety monitoring Device Compatibility • Compatible with major laptop brands: MacBook, Dell, Lenovo, HP, Surface • Works with tablets, smartphones, handheld consoles, drones, and USB-C accessories • Supports Power Delivery 3.1 for next-generation fast charging Build & Design • Dimensions: 57 × 49.7 × 161.7 mm • Weight: ~665 g • Premium aluminum finish with integrated digital display • Shows real-time wattage, battery percentage, and port activity Safety & Protection • Active thermal regulation • Surge, overload, and short-circuit protection • Smart voltage control for sensitive devices | 4 Pcs |
| 42 | Drum Unit for bizhub C258 | Drum for Konica Minolta bizhub C258 Black Drum Unit | 4 Pcs |
| 43 | Drum Units (C / M / Y) for bizhub C258 | Drum for Konica Minolta bizhub C258 Color Drum Units (C / M / Y) | 4 Pcs |
| 44 | Waste Toner for Konica Minolta bizhub C258 | Waste Toner for Konica Minolta bizhub C258 | 2 Pcs |
| 45 | Plotter INK for HP DesignJet T930 | Plotter INK for HP DesignJet T930, | 2 Set |
| 46 | Toner black for Konica Minolta C258 | Toner black for Konica Minolta C258, | 2 Pcs |
| 47 | Toner cyan for Konica Minolta C258 | Toner cyan for Konica Minolta C258 | 2 Pcs |
| 48 | Toner magenta for Konica Minolta C258 | Toner magenta for Konica Minolta C258 | 2 Pcs |
| 49 | Toner Yellow for Konica Minolta C258 | Toner Yellow for Konica Minolta C258 | 2 Pcs |
| 50 | Drum for Sharp DX-2500 | Drum for Sharp DX-2500 | 2 Set |
| 51 | Toner black, for Sharp DX-2500 | Toner black, for Sharp DX-2500 | 2 Pcs |
| 52 | Toner cyan, for Sharp DX-2500 | Toner cyan, for Sharp DX-2500 | 2 Pcs |
| 53 | Toner magenta, for Sharp DX-2500 | Toner magenta, for Sharp DX-2500 | 2 Pcs |
| 54 | Toner Yellow, for Sharp DX-2500 | Toner Yellow, for Sharp DX-2500 | 2 Pcs |
| 55 | Toner for Cannon Pixma black | Toner for Cannon Pixma black | 8 Pcs |
| 56 | Toner for Cannon Pixma Colored | Toner for Cannon Pixma Colored | 2 Pcs |
| 57 | Microsoft Windows 11 Enterprise | 1year subscription | 30 Lic |
| 58 | Microsoft Entra ID P2 | 1year subscription | 30 Lic |
| 59 | Microsoft Defender for Endpoint | 1year subscription | 30 Lic |
| 60 | Microsoft 365 E3 | 1year subscription | 30 Lic |
| 61 | Microsoft 365 E5 | 1year subscription | 30 Lic |
| 62 | Power BI | 1year subscription | 10 Lic |
| 63 | Adobe Acrobat Pro | 1year subscription | 2 Lic |
| 64 | Copilot Studio | 1year subscription | 1 Lic |
| 65 | Canva Business | 1year subscription | 3 Lic |
| 66 | Photoshop beta 2026 | 1year subscription | 2 Lic |
| 67 | Google Flow | 1year subscription | 1 Lic |
| 68 | AutoCAD 2026-Architecture | 1year subscription | 2 Lic |
| 69 | Sketchup | 1year subscription | 2 Lic |
| 70 | Lumion 2026 software | 1year subscription | 1 |

Very Important:

Note (1): No technical data sheets are required for ink, toner, and drum items. However, the Supplier shall ensure that all quoted items are original products and not compatible or refurbished alternatives.

The Bidder must submit a signed and stamped declaration confirming that all items listed under Item Nos. 42 to 56 are original products only, and not compatible, remanufactured, or third-party equivalents. This declaration shall be included and submitted together with the technical documentation/data sheets.

Note (2): The Bidder shall submit a signed and stamped declaration confirming that all IT hardware and equipment are covered by a minimum one (1) year warranty from the date of delivery and acceptance.