## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest-cost technically compliant bid

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid, rather than the total price without go back to the supplier. The unit price must be provided with no more than two decimal places.

- Unit price must include, labor, packaging, transport, delivery to LRC warehouse, and all applicable bank transfer charges

- The bidder is allowed to propose only one option for each item.

Lot #1: Skid Steer & Accessories Submission of technical datasheets for each proposed item is mandatory.

Note: All skid steer accessories must be fully compatible with the proposed skid steer unit. Preference shall be given to same-brand equipment to ensure full operational integration, safety, and performance compliance.

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item # | Item/Milestone Required | Unit of Measurement (UOM) | QTY | Unit Price in USD Exclusive of VAT | VAT (%) | Unit Price in USD Inclusive of VAT | Total Price in USD Inclusive of VAT | Lead time of delivery (in calendar days) from the date of each Purchase Order. |
| 1 | Skid Steer | Unit | 1 |  |  |  |  |  |
| ## 2. Skid Steer Accessories |  |  |  |  |  |  |  |  |
| 2.1 | Buckets & Material Handling |  |  |  |  |  |  |  |
| 2.1.1 | Construction Bucket | Unit | 1 |  |  |  |  |  |
| 2.1.2 | Skeleton Bucket | Unit | 1 |  |  |  |  |  |
| 2.1.3 | Grapple | Unit | 1 |  |  |  |  |  |
| 2.1.4 | Pallet Forks (Hydraulic) | Unit | 1 |  |  |  |  |  |
| 2.1.5 | Utility Frame | Unit | 1 |  |  |  |  |  |
| 2.2 | Excavation & Groundwork |  |  |  |  |  |  |  |
| 2.2.1 | Backhoe | Unit | 1 |  |  |  |  |  |
| 2.2.2 | Breaker | Unit | 1 |  |  |  |  |  |
| 2.2.3 | Rear Stabilizers | Unit | 1 |  |  |  |  |  |
| 2.3 | Cleaning & Maintenance |  |  |  |  |  |  |  |
| 2.3.1 | Sweeper | Unit | 1 |  |  |  |  |  |
| 2.3.2 | Pressure Washer | Unit | 1 |  |  |  |  |  |
| 2.3.3 | Angle Broom | Unit | 1 |  |  |  |  |  |
| 2.3.4 | Water Kit | Unit | 1 |  |  |  |  |  |
| 2.4 | Concrete Equipment |  |  |  |  |  |  |  |
| 2.4.1 | Concrete Mixer | Unit | 1 |  |  |  |  |  |
| 2.4.2 | Mixing Bucket | Unit | 1 |  |  |  |  |  |
| 2.4.3 | Concrete Pump | Unit | 1 |  |  |  |  |  |

Lot #2: Counterbalance Diesel 3.5t Submission of technical datasheets for each proposed item is mandatory.

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item # | Item/Milestone Required | Unit of Measurement (UOM) | Estimated Quantity / 2 Year | Unit Price in USD Exclusive of VAT | VAT (%) | Unit Price in USD Inclusive of VAT | Total Price in USD Inclusive of VAT | Lead time of delivery (in calendar days) from the date of each Purchase Order. |
| 1 | Counterbalance Diesel 3.5t | Unit | 1 |  |  |  |  |  |
| 2 | Forks 160 cm compatible with the Item #1 | Pair | 1 |  |  |  |  |  |

## ANNEX 3: DETAILED SPECIFICATIONS

| ITEM # | ITEM NAME | Technical Specifications |  | QTY / UOM |
| --- | --- | --- | --- | --- |
| Lot #1: Skid Steer & Accessories Submission of technical datasheets for each proposed item is mandatory. |  |  |  |  |
| 1 | Skid Steer | Performance Rated operating capacity: 894 kg Tipping load: 1,788 kg Lift Type: Radial Lift Engine: Rated power: 60-62 hp Maximum torque: 201 Nm Configuration: Turbocharged with direct injection diesel Cooling system: Water-cooled with Intercooler Transmission Type: Hydrostatic Gearbox: 2-speed forward/reverse Performance & Mobility Drive: 4 Wheel Drive Maximum travel speed: 17 km/h Controls: Mechanical Hydraulics: High flow Features: Closed Cabin with HVAC |  | 1 Unit |
| ## 2. Skid Steer Accessories The proposed accessories shall be fully compatible with the skid steer loader, matching the same brand specifications, technical requirements, and operational standards to ensure safe and efficient operation. |  |  |  |  |
| 2.1 Buckets & Material Handling |  |  |  |  |
| 2.1.1 | Construction Bucket | Capacity: 0.5 m³ Width: 1830 mm Weight: 320 kg Bolt-on cutting edge |  | 1 Unit |
| 2.1.2 | Skeleton Bucket | Width: 1800 mm Weight: 280 kg Bar spacing: 75 mm Capacity: 0.45 m³ |  | 1 Unit |
| 2.1.3 | Grapple | Jaw opening: 900 mm Clamping force: 25 kN Width: 1700 mm Weight: 420 kg |  | 1 Unit |
| 2.1.4 | Pallet Forks (Hydraulic) | Capacity: 2268 kg Fork length: 1220 mm Width: 1219 mm Weight: 256 kg |  | 1 Unit |
| 2.1.5 | Utility Frame | Capacity: 1500 kg Width: 1200 mm Weight: 180 kg |  | 1 Unit |
| 2.2 Excavation & Groundwork |  |  |  |  |
| 2.2.1 | Backhoe | Dig depth: 1.83 m Swing angle: 48° Weight: 286 kg |  | 1 Unit |
| 2.2.2 | Breaker | Impact energy: 750–1000 J class Flow: 60–80 L/min Weight: ~500 kg |  | 1 Unit |
| 2.2.3 | Rear Stabilizers | Downforce: 30 kN Width: 2200 mm Weight: 300 kg |  | 1 Unit |
| 2.3 Cleaning & Maintenance |  |  |  |  |
| 2.3.1 | Sweeper | Width: 1700 mm Hopper: 0.4 m³ Brush diameter: 650 mm Weight: 480 kg |  | 1 Unit |
| 2.3.2 | Pressure Washer | Pressure: 200 bar Flow: 15 L/min Tank: 200 L Weight: 220 kg |  | 1 Unit |
| 2.3.3 | Angle Broom | Width: 1600–1800 mm Brush diameter: 600 mm Angle: ±30° Weight: 350 kg |  | 1 Unit |
| 2.3.4 | Water Kit | Tank: 300 L Pump flow: 20 L/min Weight: 180 kg |  | 1 Unit |
| 2.4 Concrete Equipment |  |  |  |  |
| 2.4.1 | Concrete Mixer | Capacity: 0.35 m³ Output: 300 L Weight: 400 kg |  | 1 Unit |
| 2.4.2 | Mixing Bucket | Capacity: 0.4 m³ Auger speed: 25 rpm Weight: 450 kg |  | 1 Unit |
| 2.4.3 | Concrete Pump | Output: 15–20 m³/hr Pressure: up to 60 bar Weight: 650 kg |  | 1 Unit |
| Lot #2 - Counterbalance Diesel 3.5t Submission of technical datasheets for each proposed item is mandatory. |  |  |  |  |
| 1 | Counterbalance Diesel 3.5t |  | ### Needed Qualifications: Durable, Parts Availability, Fuel Efficient, Made in USA Japan or EU Technical Feature Engine: Diesel Max lift height: 4500mm Triplex Max capacity: 3000kg Mode: Diesel Transmission Type: Hydrostatic Lift Type; Free Lift Tank Capacity: 50 L Side Shifter: Hydraulic Wheels: Double Rim Safety Features Brakes: Hydraulics Include: Seatbelt, Overhead Guard, Horns and Lights, Load Backrest and Emergency stop button | 1 Unit |
| 2 | Fork 160 cm |  | ### Forks 160 cm fully compatible with Counterbalance Diesel 3.5T forklift as per the Item #1 (LOT 3) Annex 3 - specifications. | 1 Pairs |