## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest evaluated technically compliant bidder meeting all administrative, technical, and contractual requirements.

- The quantities indicated are estimated for evaluation purposes. The contract shall be awarded based on the scope defined in the Terms of Reference and the final quantities confirmed by LRC. Any variation to the quantities or scope shall be subject to mutual written agreement between the parties through a contract amendment.

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid rather than the total price, without referring the matter back to the supplier for clarification. Unit prices shall be provided with no more than two (2) decimal places.

- The unit prices quoted shall be deemed to include all direct and indirect costs associated with the performance of the contract, including equipment, accessories, installation materials, cabling, configuration, commissioning, testing, training, warranty obligations, technical support, transportation, labor, overheads, and any other costs necessary for the successful execution of the project.

| LRC To Complete |  |  |  | Bidder to Complete |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item # | Item/Milestone Required | UNIT | Estimated Quantity | Unit Price in USD Exclusive of VAT | VAT (%) | Unit Price in USD Inclusive of VAT | Total Price in USD Inclusive of VAT | Implementation Period (Calendar Days) from Contract Signature |
| 1 | UHF digital/analog base repeater, NXR-5800 class | Unit | 3 |  |  |  |  |  |
| 2 | 19-inch rack mounting kit, rails, and brackets for repeaters | Set | 3 |  |  |  |  |  |
| 3 | 3-channel UHF RF combining/filtering system | Set | 1 |  |  |  |  |  |
| 4 | AC/DC power supply for each repeater | Unit | 3 |  |  |  |  |  |
| 5 | Power supply for RF combiner/multicoupler system | Unit | 1 |  |  |  |  |  |
| 6 | 19-inch floor-standing waterproof rack cabinet, minimum 18U | Unit | 1 |  |  |  |  |  |
| 7 | Rack PDU with breaker and surge protection | Unit | 1 |  |  |  |  |  |
| 8 | Rack fan tray or ventilation unit | Unit | 1 |  |  |  |  |  |
| 9 | Rack grounding bar and grounding accessories | Lot | 1 |  |  |  |  |  |
| 10 | Low-loss RF jumper cables between repeaters and combiner | Lot | 1 |  |  |  |  |  |
| 11 | Low-loss RF feeder cable from combiner to antenna system | Lot | 1 |  |  |  |  |  |
| 12 | N-type RF connectors, adaptors, and accessories | Lot | 1 |  |  |  |  |  |
| 13 | RF lightning arrestor | Unit | 1 |  |  |  |  |  |
| 14 | UHF outdoor antenna, mast/bracket, and mounting accessories | Set | 1 |  |  |  |  |  |
| 15 | Cat6 network patch cables | Lot | 1 |  |  |  |  |  |
| 16 | Cable management accessories | Lot | 1 |  |  |  |  |  |
| 17 | Online UPS or DC backup power system | Unit | 1 |  |  |  |  |  |
| 18 | Programming and configuration service | Lot | 1 |  |  |  |  |  |
| 19 | RF tuning and optimization | Lot | 1 |  |  |  |  |  |
| 20 | Installation and commissioning | Lot | 1 |  |  |  |  |  |
| 21 | Testing and acceptance report | Lot | 1 |  |  |  |  |  |
| 22 | Training for LRC technical team | Session | 1 |  |  |  |  |  |
| 23 | As-built documentation | Lot | 1 |  |  |  |  |  |
| 24 | Warranty and technical support | Year | 3 |  |  |  |  |  |

ANNEX 3: TERMS OF REFERENCE (ToR)

Subject: Procurement, Installation, Configuration, Commissioning and Three-Year Maintenance of a UHF Repeater System for the Lebanese Red Cross # 1. Executive Summary The Lebanese Red Cross (LRC) intends to select a qualified supplier to supply, install, configure, test, commission, document, and support a complete UHF repeater system for operational and emergency radio communications. The required solution shall be delivered as a turnkey system. The supplier shall not limit the offer to standalone radio devices. The final solution must include the repeaters, RF combining/filtering system, rack cabinet, RF cabling, power distribution, grounding, configuration, commissioning, training, warranty, and technical support required to make the system fully operational. The system shall be similar in function and operational capacity to the existing LRC UHF repeater setup, while allowing equivalent or better professional-grade equipment, provided that interoperability, reliability, maintainability, and regulatory compliance are fully met. ## 1.1 Strategic Outcomes Reliable UHF radio communication for LRC operational, emergency, field, and dispatch activities. A clean, secure, rack-mounted, maintainable repeater site installation. Interoperability with the existing LRC UHF radio operating procedures. A documented and supportable configuration baseline for future maintenance and expansion. Reduced operational risk through proper RF design, grounding, lightning protection, backup power, testing, and handover. # 2. Background and Current Environment LRC relies on radio communication to support time-sensitive operations, field coordination, dispatch coordination, and continuity of service during emergencies. The current UHF repeater environment includes rack-mounted digital/analog repeaters, a 3-channel UHF RF combining/filtering system, a 19-inch rack installation, and associated RF, power, and cabling infrastructure. The existing system reference shows three rack-mounted UHF repeaters, a dedicated 3-channel UHF RF combiner/filtering unit, RF interconnection cabling, power distribution, and rack installation. The new tender shall procure a solution that provides equivalent or better technical capability and professional implementation quality. # 3. Assignment Objectives Supply professional-grade UHF digital/analog repeater equipment suitable for continuous mission-critical operation. Provide a complete 3-channel UHF repeater site configuration unless otherwise specified by LRC. Provide RF combining/filtering equipment suitable for the approved LRC frequency plan. Install all equipment in a proper 19-inch rack cabinet with clean cabling, grounding, ventilation, and power protection. Configure all channels, analog/digital parameters, network settings, and security settings based on LRC requirements. Tune, test, and commission the full RF system and provide documented test results. Train LRC technical staff and hand over all final documentation, credentials, configuration backups, and warranty records. # 4. Scope of Work The selected supplier shall be responsible for end-to-end implementation. The scope includes all equipment, accessories, professional services, configuration, testing, documentation, and support needed to deliver a fully operational UHF repeater system. ## 4.1 Minimum Scope Components Conduct a technical site survey and validate installation requirements. Review the existing UHF repeater setup and final operational requirements with LRC. Prepare the final technical design, RF design, rack layout, and implementation plan. Supply all repeaters, RF combiner/filtering components, rack, power, cables, connectors, labels, grounding materials, and accessories. Install all equipment professionally in the rack cabinet. Program and configure the repeaters according to the approved LRC frequency and channel plan. Tune RF combiner, duplexer/filtering, RX multicoupler, preselector, and antenna path where applicable. Perform full functional, RF, power, coverage, and acceptance testing. Deliver documentation, configuration files, test reports, training, and final handover. Provide warranty and post-implementation support according to this ToR. Turnkey responsibility Any item required to make the system operational, stable, safe, compliant, and maintainable shall be considered included in the supplier offer, even if not explicitly listed item by item in this ToR or BOQ.. # 5. Existing System Reference and Design Assumptions Reference Area; Design Assumption. Repeater class; Rack-mounted UHF digital/analog base repeater, Kenwood NEXEDGE NXR-5700/5800 class, The proposed solution shall provide equivalent or superior functionality, performance, interoperability, and reliability.. Site capacity; Three UHF repeater channels per site as baseline.. RF combining/filtering; 3-channel UHF combining/filtering system similar to Amphenol Procom PRO-COM450-HDAR-3/H class, The proposed solution shall provide equivalent or superior technical performance and compatibility with the required operational environment.. Rack setup; 19-inch rack cabinet with repeaters, RF combiner/filtering unit, power equipment, and RF interconnection cabling.. Frequency handling; Actual LRC TX/RX frequencies, tones, digital IDs, and site parameters are confidential and shall be shared only under controlled conditions.. Reuse of existing infrastructure; Existing antenna, feeder, rack, power, or grounding may be reused only after site survey validation and written approval by LRC.. # Note: Brand names and model references are provided solely to describe the existing LRC system and the required level of functionality and performance. Bidders may propose equivalent or superior solutions that fully comply with the requirements of this ToR. # 6. Minimum Technical Specifications ## 6.1 UHF Digital/Analog Repeater Requirement; Minimum Specification. Type; Professional UHF digital/analog base repeater.. Reference class; Kenwood NEXEDGE NXR-5800 class or equivalent.. Quantity; Minimum 3 repeaters per site unless otherwise specified in the BOQ.. Frequency band; Must support LRC licensed UHF frequencies within the UHF band, including 400-470 MHz / 406-470 MHz range as applicable.. RF output power; Minimum 25 W RF output; selectable output power preferred.. Duty cycle; 100% continuous duty cycle at rated output power.. Channel capacity; Minimum 30 programmable channels per repeater.. Channel spacing; Must support 6.25 kHz and 12.5 kHz digital operation; 12.5 kHz and 25 kHz analog operation where legally permitted.. Digital mode; NXDN digital air interface or fully compatible digital standard accepted by LRC.. Analog mode; Conventional FM analog operation.. Mixed mode; Must support analog/digital mixed-mode operation.. Operation mode; Repeater mode and base station mode.. Frequency stability; Plus/minus 0.5 ppm or better; OCXO option preferred.. Receiver sensitivity; Professional-grade sensitivity suitable for mission-critical UHF repeater operation. Bidders shall state exact sensitivity values in the compliance matrix.. Interfaces; Ethernet LAN, USB programming interface, auxiliary I/O, and external controller interface.. Monitoring indicators; LED indicators for power, transmit, busy, status/alarm, and channel status.. Remote management; Remote diagnostics, programming backup, and firmware update capability preferred.. Network support; IP/LAN connectivity for monitoring and future networked operation.. Audio; Internal or external speaker support.. Power input; Suitable DC input with AC/DC power supply for 220-240 VAC, 50 Hz.. Mounting; 19-inch rack-mountable, maximum 1U per repeater preferred.. Operating temperature; Minimum -30°C to +60°C or equivalent professional operating range.. Regulatory compliance; Must comply with applicable Lebanese telecom/radio licensing requirements and relevant CE/ETSI or equivalent standards.. ## 6.2 RF Combining and Filtering System Requirement; Minimum Specification. Type; 3-channel UHF RF combining/filtering system.. Reference class; Amphenol Procom PRO-COM450-HDAR-3/H class or equivalent.. Quantity; 1 complete system per site.. Frequency range; Must support assigned LRC UHF operating frequencies.. Channel support; Minimum 3 transmit channels.. Main components; TX hybrid combiner, duplex filter, active RX multicoupler, preselector, TX isolators, and all required RF accessories.. RX ports; Minimum 4 RX output ports.. Maximum input power; Minimum 50 W per TX input.. Duplex spacing; Must support LRC licensed duplex spacing.. TX-TX spacing; Must support current and future LRC channel spacing within the same UHF site.. TX path loss; Low-loss design; less than 7 dB preferred. Bidder shall declare actual expected loss.. TX-TX isolation; Minimum 70 dB.. Antenna-TX isolation; Minimum 40 dB.. RX-RX isolation; Minimum 20 dB.. VSWR; Less than 1.5:1 preferred.. Impedance; 50 Ohms.. Connectors; N-type female or equivalent professional RF connectors.. Tuning; Factory tuned or tuned by certified RF technician to LRC frequencies.. Test report; Supplier shall provide tuning, insertion loss, isolation, return loss, and VSWR reports.. ## 6.3 Antenna System Requirement; Minimum Specification. Antenna type; Professional outdoor UHF antenna suitable for repeater site operation.. Frequency range; Must cover the assigned LRC UHF frequency band.. Impedance; 50 Ohms.. Connector; N-type female or equivalent professional RF connector.. Mounting; Mast, brackets, clamps, stainless accessories, and safe installation accessories as required.. Protection; Outdoor connector weatherproofing and RF lightning arrestor at entry point.. Validation; Final antenna choice must be supported by VSWR and coverage testing results.. # 7. Configuration and Programming Requirements ## 7.1 Frequency and Channel Programming Program all TX/RX frequencies based on the official LRC frequency plan. Configure channel names and display labels as requested by LRC. Configure analog parameters including CTCSS/DCS, squelch, hang time, time-out timer, CW ID, and audio levels. Configure digital parameters including RAN, group IDs, unit IDs, site parameters, and access permissions where applicable. Configure repeater behavior for analog, digital, and mixed-mode operation. Ensure compatibility with existing LRC radios or clearly identify any required radio-side reprogramming before implementation. ## 7.2 Network and Management Configuration Assign static IP addresses where IP management is required. Configure VLAN, subnet, gateway, and remote monitoring settings if required by LRC. Change all default passwords before handover. Provide credentials to LRC in a secure and documented manner. Provide final programming files and configuration backup files for all repeaters and management components. Enable remote diagnostics and monitoring where supported and approved by LRC. ## 7.3 Confidentiality of Frequencies and Configuration The final TX/RX frequencies, tones, digital IDs, channel plan, site parameters, programming files, and passwords shall be treated as confidential. Bidders shall not publish, disclose, reuse, or share this information with third parties without written LRC approval. # 8. Installation, Rack, Cabling, Power and Grounding Requirements ## 8.1 Rack Cabinet and Physical Installation Requirement; Minimum Specification. Rack cabinet; 19-inch waterproof floor-standing rack cabinet.. Rack size; Minimum 18U, or larger if required by final design.. Rack depth; Minimum 600 mm; 800 mm preferred where space allows.. Access; Lockable front and rear access; removable side panels preferred.. Ventilation; Fan tray or forced ventilation suitable for continuous repeater operation.. Mounting; All equipment shall be securely mounted using appropriate rails and brackets.. Cable management; Horizontal and vertical cable managers, Velcro ties, labels, and organized cable routing.. Labelling; All equipment, ports, RF cables, power cables, network cables, and channels shall be clearly labelled.. Clean installation; No loose cables, exposed conductors, unsupported RF cables, or temporary wiring shall be accepted.. ## 8.2 Power and Backup Requirements Requirement; Minimum Specification. AC power; 220-240 VAC, 50 Hz.. Power distribution; Rack-mounted PDU with breaker and surge protection.. DC power; Suitable DC power supplies for repeaters and RF equipment.. Backup power; UPS or DC backup system shall be proposed as part of the solution. Autonomy shall be stated by bidder.. Grounding; Proper electrical grounding and RF bonding shall be implemented.. Protection; Surge protection and RF lightning protection shall be included where antenna/feedline is connected.. ## 8.3 RF, Network, and Internal Cabling Requirement; Minimum Specification. RF jumper cables; 50 Ohm low-loss RF jumpers between repeaters, combiner, duplexer/filtering unit, and antenna system.. Feeder cable; Low-loss outdoor RF feeder cable suitable for UHF operation; final type and length to be confirmed by site survey.. Connectors; Professional N-type connectors or equivalent.. Weatherproofing; Outdoor RF connections shall be weatherproofed.. Lightning protection; RF lightning arrestor at antenna cable entry point.. Network cabling; Cat6 patch cables for LAN/monitoring connections.. Cable labels; All cables shall be labelled at both ends.. Cable routing; RF, DC, AC, and data cables shall be routed separately where possible.. # 9. Testing, Commissioning and Acceptance The supplier shall perform and document all testing activities before final handover. Final acceptance shall be granted only after LRC confirms that all mandatory tests, documentation, training, and corrective actions are completed. Test; Minimum Acceptance Requirement. Physical installation test; Equipment securely mounted, labelled, grounded, ventilated, and accessible for maintenance.. Power test; All equipment powered correctly through approved power distribution and backup source where applicable.. RF output test; Each repeater tested at configured RF output power.. Receiver test; RX performance and reception verified.. VSWR test; Antenna/feedline VSWR less than 1.5:1 preferred.. Combiner/filter test; Insertion loss, isolation, tuning, return loss, and frequency alignment tested and reported.. Analog communication test; Clear analog communication with existing LRC radios.. Digital communication test; Clear digital communication with existing LRC radios where digital mode is required.. Mixed-mode test; Analog/digital mixed-mode operation verified where required.. Coverage test; Field test from agreed operational locations.. Interference test; No harmful interference between channels.. Network test; LAN/monitoring access verified where applicable.. Backup power test; UPS/DC backup operation verified where included.. Restart and recovery test; System restart and service recovery verified.. Documentation review; Configuration files, diagrams, reports, credentials, manuals, and warranty records submitted.. # 10. Deliverables Site survey before installation in Kabr Chemoun location Complete equipment supply and installation. Final repeater programming and configuration. RF tuning and optimization report. Testing and commissioning report. Frequency/channel programming summary without exposing confidential details in public documentation. Rack layout diagram. RF cabling diagram. Power and grounding diagram. Network configuration summary, where applicable. Equipment datasheets and manuals. Configuration backup files. Warranty certificates. Training material and attendance sheet. As-built documentation. Support escalation matrix. # 11. Training and Knowledge Transfer The supplier shall provide practical training to designated LRC technical staff. Training shall be delivered after commissioning and before final acceptance, unless otherwise agreed by LRC. System architecture and main components. Repeater operation and front-panel indicator interpretation. Basic troubleshooting and safe restart procedures. Configuration backup and restore process. RF cable, connector, grounding, and lightning protection precautions. Basic preventive maintenance checklist. # 12. Warranty and Support Requirements The supplier shall provide a minimum warranty period of three years from the date of final acceptance. Warranty shall cover hardware defects, supplied RF components, power supplies, installation workmanship, supplier configuration issues, firmware support where applicable, and technical assistance. Priority; Description; Required Response. Critical; Total system outage or major operational impact.; Response within 4 hours.. High; One channel/repeater unavailable or major degradation.; Response within 8 hours.. Medium; Degraded performance or non-critical fault.; Response within 1 business day.. Low; Documentation, configuration, minor change, or advisory support.; Response within 2 business days.. The bidder shall clearly state whether support is available 24/7, during business hours, or under a separate support package. Any exclusions must be explicitly listed in the technical and financial offers. Response time shall mean acknowledgement of the issue and commencement of corrective action, whether remotely or on-site, as appropriate. # 13. Bidder Qualifications and Mandatory Eligibility Bidders must demonstrate technical experience in professional UHF/VHF radio repeater systems, RF engineering, installation, tuning, and post-installation support. Company profile and legal registration documents. Proof of experience in UHF/VHF repeater systems. At least three similar project references. Manufacturer or authorized distributor letter where applicable. Profiles of assigned RF engineers/technicians. Proof of capacity to program, tune, test, and commission repeater systems. Availability of RF test tools such as RF service monitor, power meter, spectrum analyzer, VSWR meter, dummy load, and related tools. Confirmation of after-sales support capacity in Lebanon. Warranty and support statement. # 14. Bid Submission Requirements ## 14.1 Technical Offer - The bidder shall submit a complete Technical Proposal containing, at a minimum, the following documents: Proposed solution design and architecture. Compliance matrix completed against this ToR. Datasheets for all proposed equipment. Proposed rack layout. Proposed RF diagram. Proposed power and backup design. Implementation methodology and project timeline. Testing and commissioning plan. Training plan. Warranty and support details. Assumptions, exclusions, dependencies, and optional items. - Failure to submit any mandatory technical document or information requested under this ToR may result in the bid being declared non-responsive and excluded from further technical evaluation. ## 14.2 Financial Offer Itemized BOQ with unit prices and total prices. Installation cost. Configuration and programming cost. RF tuning and commissioning cost. Training cost. Warranty and support cost. Optional items clearly separated. Offer validity period. Delivery and implementation timeline. All taxes, shipping, customs, and site-related costs clearly stated. Mandatory rejection principle LRC reserves the right to reject any offer that does not meet mandatory technical, interoperability, regulatory, confidentiality, or support requirements, even if it is the lowest financial offer.. # 15. LRC Responsibilities and Dependencies Nominate LRC technical focal points for coordination, site access, testing, and acceptance. Provide approved frequency plan and configuration requirements under confidentiality. Provide access to existing site infrastructure for survey and installation. Review and approve final design before implementation. Coordinate any required internal approvals, site access, and security clearance. Confirm whether existing antenna, feeder, rack, power, or grounding can be reused after supplier survey. Participate in testing, training, and final acceptance activities. # 16. Bill of Quantities The below BOQ is for one 3-channel UHF repeater site. Bidders shall price each line item clearly and may add optional items separately where needed. Final quantities shall be confirmed by LRC in the tender package. #; Item Description; Unit; Qty; Remarks. 1; UHF digital/analog base repeater, NXR-5800 class; Unit; 3; Minimum 25 W, 100% duty cycle, NXDN/FM or LRC-approved equivalent.. 2; 19-inch rack mounting kit, rails, and brackets for repeaters; Set; 3; If not included with repeater.. 3; 3-channel UHF RF combining/filtering system; Set; 1; Including TX combiner, duplex filter, RX multicoupler, preselector, isolators, and required accessories.. 4; AC/DC power supply for each repeater; Unit; 3; Sized according to repeater power requirement.. 5; Power supply for RF combiner/multicoupler system; Unit; 1; If required and not included.. 6; 19-inch floor-standing waterprrof rack cabinet, minimum 18U; Unit; 1; Lockable, ventilated, with side panels and front/rear access.. 7; Rack PDU with breaker and surge protection; Unit; 1; 220-240 VAC.. 8; Rack fan tray or ventilation unit; Unit; 1; Suitable for continuous operation.. 9; Rack grounding bar and grounding accessories; Lot; 1; Including bonding cables.. 10; Low-loss RF jumper cables between repeaters and combiner; Lot; 1; 50 Ohm, N-type connectors.. 11; Low-loss RF feeder cable from combiner to antenna system; Lot; 1; Length to be confirmed after site survey.. 12; N-type RF connectors, adaptors, and accessories; Lot; 1; As required.. 13; RF lightning arrestor; Unit; 1; Installed at antenna cable entry point.. 14; UHF outdoor antenna, mast/bracket, and mounting accessories; Set; 1. 15; Cat6 network patch cables; Lot; 1; For monitoring/management.. 16; Cable management accessories; Lot; 1; Cable managers, Velcro ties, labels, cable sleeves, and routing accessories.. 17; Online UPS or DC backup power system; Unit; 1; Autonomy to be proposed by bidder.. 18; Programming and configuration service; Lot; 1; Channels, IDs, tones, digital settings, network settings, passwords, and backups.. 19; RF tuning and optimization; Lot; 1; Combiner, filters, duplexer, RX/TX path, feeder and antenna path.. 20; Installation and commissioning; Lot; 1; Complete site implementation.. 21; Testing and acceptance report; Lot; 1; RF, operational, power, coverage, and network tests where applicable.. 22; Training for LRC technical team; Session; 1; Practical training and handover.. 23; As-built documentation; Lot; 1; Diagrams, configuration backup, manuals, serial list, support contacts.. 24; Warranty and technical support; Year; 3; Three-year warranty and technical support including preventive maintenance, corrective maintenance, remote support, and on-site intervention as required. # 17. Annex D - Bidder Confidentiality Declaration The bidder confirms that any LRC information related to radio frequencies, channel plans, tones, digital IDs, radio IDs, serial numbers, site locations, network details, passwords, configuration files, coverage information, and operational procedures shall be treated as confidential and shall not be disclosed, copied, reused, published, or transferred to any third party without prior written approval from LRC. Bidder Name: _______________________________________________ Authorized Representative: __________________________________ Signature and Company Stamp: ________________________________ Date: _______________________________________________________