ITB Reference No.: ITB/2026-051
Subject: Procurement, Installation, Configuration, Commissioning and Three-Year Maintenance of a UHF Repeater System for the Lebanese Red Cross

Bidders must complete this compliance matrix and submit it with the technical offer. LRC may request supporting datasheets, test reports, demonstrations, or clarifications for any requirement.

| # | Requirement | Compliant / Partially Compliant / Not Compliant | Bidder Response | Reference Page / Datasheet |
| --- | --- | --- | --- | --- |
| 1 | UHF digital/analog repeater offered |  |  |  |
| 2 | Minimum 3 repeaters per site included |  |  |  |
| 3 | 25 W minimum RF output supported |  |  |  |
| 4 | 100% duty cycle supported |  |  |  |
| 5 | NXDN or LRC-approved compatible digital operation supported |  |  |  |
| 6 | Analog FM operation supported |  |  |  |
| 7 | Mixed analog/digital mode supported |  |  |  |
| 8 | 6.25 kHz and 12.5 kHz digital channel spacing supported |  |  |  |
| 9 | 12.5 kHz analog channel spacing supported |  |  |  |
| 10 | 19-inch rack mounting included |  |  |  |
| 11 | Ethernet/LAN interface available |  |  |  |
| 12 | USB programming interface available |  |  |  |
| 13 | Remote diagnostics or monitoring available |  |  |  |
| 14 | 3-channel RF combiner/filtering system included |  |  |  |
| 15 | TX combiner, duplex filter, RX multicoupler, preselector, and isolators included |  |  |  |
| 16 | Minimum 50 W input power per TX channel supported |  |  |  |
| 17 | Minimum 70 dB TX-TX isolation supported |  |  |  |
| 18 | VSWR less than 1.5:1 preferred |  |  |  |
| 19 | Professional RF cabling and N-type connectors included |  |  |  |
| 20 | RF lightning protection included |  |  |  |
| 21 | Water proof Rack cabinet included |  |  |  |
| 22 | Rack grounding and RF bonding included |  |  |  |
| 23 | UPS or backup power system proposed |  |  |  |
| 24 | Programming and configuration included |  |  |  |
| 25 | RF tuning included |  |  |  |
| 26 | Testing and commissioning included |  |  |  |
| 27 | Training included |  |  |  |
| 28 | As-built documentation included |  |  |  |
| 29 | Minimum 3 years warranty included |  |  |  |
| 30 | Support response times accepted |  |  |  |

We hereby certify that the information provided above is true and accurate, and we agree to provide the required services in full compliance with LRC’s Terms of Reference.

| Company Name: |  |
| --- | --- |
| Authorized Representative (Name & Title): |  |
| Signature & Stamp: |  |
| Date: |  |