## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest-cost technically compliant complete lot or complete bid.

- The quantities mentioned below are required unless there is a variation within a specified range with proper justification. The LRC reserves the right to increase or decrease the quantities.

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid, rather than the total price without go back to the supplier. The unit price must be provided with no more than two decimal places.

- Only one option is allowed for biding

- The bidder shall price each of the three distribution-line schedules separately. All unit rates shall include labor, materials, equipment, transport, testing, permits, overheads, incidental works, and all obligations necessary for the complete execution and handover of the works.

LOT 1:  Zahle PHC

| LRC To Complete |  |  |  | Bidder To Complete |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT | VAT (%) | Unit Price in USD, inclusive VAT | Total Price in USD, inclusive VAT |
| Division 1 |  |  |  |  |  |  |  |
| 1 | Preliminaries & General Requirements |  |  |  |  |  |  |
| 1.1 | Mobilization & Demobilization | LS | 1 |  |  |  |  |
| 1.2 | Site Establishment | LS | 1 |  |  |  |  |
| 1.3 | Temporary Utilities | LS | 1 |  |  |  |  |
| 1.4 | Temporary Fencing & Signage | LS | 1 |  |  |  |  |
| 1.5 | Traffic Management and Safety Barriers | LS | 1 |  |  |  |  |
| 1.6 | Insurance Accident and liability | LS | 1 |  |  |  |  |
| 1.7 | Health & Safety Measures | LS | 1 |  |  |  |  |
| 1.8 | Site Cleaning & Waste Management | LS | 1 |  |  |  |  |
| 1.9 | Dismantling of existing elements and return to initial condition | LS | 1 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 2 |  |  |  |  |  |  |  |
| 2 | Constructions |  |  |  |  |  |  |
| 2.1 | Partitions | m² | 70 |  |  |  |  |
| 2.2 | False Ceilings | m² | 181 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 3 |  |  |  |  |  |  |  |
| 3 | Finishing |  |  |  |  |  |  |
| 3.1 | Painting for existing walls | m² | 450 |  |  |  |  |
|  | Painting for Gypsum new walls | m² | 250 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 4 |  |  |  |  |  |  |  |
| 4 | Sanitary |  |  |  |  |  |  |
| 4.1 | Sanitary fixtures | No. | 8 |  |  |  |  |
| 4.2 | Vanity top (Kitchen) | No. | 1 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 5 |  |  |  |  |  |  |  |
| 5 | Aluminum Works |  |  |  |  |  |  |
| 5.1 | Aluminum Doors | No. | 11 |  |  |  |  |
| 5.3 | Aluminum Partitions | m² | 15 |  |  |  |  |
| 5.4 | Aluminum Windows | m² | 19 |  |  |  |  |
| 5.5 | Repair and reuse existing aluminum | m² | 24 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 6 |  |  |  |  |  |  |  |
| 6 | Flooring |  |  |  |  |  |  |
| 5.1 | Floor Polishing | m² | 300 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 7 |  |  |  |  |  |  |  |
| 7 | HVAC Works |  |  |  |  |  |  |
| 7.1 | Split AC Units, 9000 BTU | No. | 7 |  |  |  |  |
| 7.2 | Split AC Units, 18000 BTU | No. | 1 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 8 |  |  |  |  |  |  |  |
| 8 | Furniture |  |  |  |  |  |  |
| 8.1 | Benche | No. | 3 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 9 |  |  |  |  |  |  |  |
| 9 | Dental Clinic |  |  |  |  |  |  |
| 9.1 | Wooden raised Platform | m² | 24 |  |  |  |  |
| 9.2 | Specialized works | LS | 1 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 10 |  |  |  |  |  |  |  |
| 10 | Electrical Works |  |  |  |  |  |  |
| 10.1 | Lighting Points (60*60) | LS | 1 |  |  |  |  |
| 10.2 | CCTV System | LS | 1 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |
| Division 11 |  |  |  |  |  |  |  |
| 11 | Wooden Elements |  |  |  |  |  |  |
| 11.1 | Existing wooden windows | U | 18 |  |  |  |  |
| Division Total |  |  |  |  |  |  | _________________ USD |

| Divisions | Items by Trade | Total Exc (VAT) | Total (Inc VAT) |
| --- | --- | --- | --- |
| Division 1 | Preliminaries & General Requirements |  |  |
| Division 2 | Constructions |  |  |
| Division 3 | Finishing |  |  |
| Division 4 | Sanitary |  |  |
| Division 5 | Aluminum Works |  |  |
| Division 6 | Flooring |  |  |
| Division 7 | HVAC Works |  |  |
| Division 8 | Furniture |  |  |
| Division 9 | Dental Clinic |  |  |
| Division 10 | Electrical Works |  |  |
| Division 11 | Wooden Elements |  |  |
| Grand Total |  |  |  |

## ANNEX 3: DETAILED SPECIFICATIONS

As per: 2026-082 Terms of Reference

| Item No | Items by trade | Description of works | Unit | Qty |
| --- | --- | --- | --- | --- |
| Division 1 |  |  |  |  |
| 1 | Preliminaries & General Requirements |  |  |  |
| 1.1 | Mobilization & Demobilization | Contractor's mobilization/demobilization: removal of personnel, equipment, tools, and temporary facilities upon completion of works | LS | 1 |
| 1.2 | Site Establishment | Contractor’s site setup, including necessary working areas, storage areas, site camp facilities (if required), and general site arrangements. | LS | 1 |
| 1.3 | Temporary Utilities | Provision, connection, maintenance, and removal of temporary water, electricity, lighting, communication, and other utilities required for the execution of works. | LS | 1 |
| 1.4 | Temporary Fencing & Signage | Provision and maintenance of temporary fencing, warning signs, safety notices, and protection measures for the work area. | LS | 1 |
| 1.5 | Traffic Management and Safety Barriers | Provision and maintenance of traffic control measures, barriers, access control, and safe circulation arrangements where required. | LS | 1 |
| 1.6 | Insurance Accident and liability | Provision of all required insurance coverage, including workers’ compensation, contractor’s liability, and third-party liability insurance. | LS | 1 |
| 1.7 | Health & Safety Measures | Provision and implementation of all necessary occupational health and safety measures throughout the execution of the works, including scaffolding, traffic management measures, safety barriers and warning signs, personal protective equipment (PPE) for all workers, safe access and working platforms, provision of drinking water, and all other safety and welfare requirements necessary to comply with applicable safety regulations and Engineer’s instructions. | LS | 1 |
| 1.8 | Site Cleaning & Waste Management | Continuous cleaning of the work areas, collection, transportation, and disposal of construction waste and debris to approved disposal locations. | LS | 1 |
| 1.9 | Dismantling of existing elements and return to initial condition | Removal of temporary facilities, fencing, utilities, and protection measures, including cleaning and reinstatement of affected areas upon completion. | LS | 1 |
| Division 2 |  |  |  |  |
| 2 | Constructions |  |  |  |
| 2.1 | Partitions | Supply, delivery, assembly, and precise installation of a 10 cm thick heavy-duty moisture-resistant gypsum board partition wall. 2. Material Specifications: Gypsum Board Enclosure: Moisture-Resistant (MR) / Waterproof-grade gypsum plasterboards. Double-layer application on both faces of the partition wall using 12.5 mm thick MR boards (yielding a total skin thickness of 25 mm per side to achieve the required structural durability and a total wall profile of ~10 cm). Metal Framing System: Hot-dip galvanized steel studs and floor/ceiling tracks (runners), sized to match the overall 10 cm partition depth. Heavy-duty structural metal framework installed with vertical studs and horizontal bridging channels positioned at maximum 40 cm centers (on-center spacing) both horizontally and vertically to form a reinforced grid. 3. Accessories & Fasteners: Corrosion-resistant, self-tapping drywall screws spaced at required centers per manufacturer guidelines for double-layer installation.Heavy-duty mechanical masonry anchors/plugs to securely fasten tracks to floor slabs, soffits, and adjacent perimeter walls. Fiberglass mesh joint tape embedded in specialized waterproof joint compound, finished flush and sanded smooth for immediate priming/painting or tiling.Perimeter Sealing: Acoustic and moisture-resistant elastomeric sealant applied along floor, ceiling, and perimeter wall tracks to prevent moisture ingress and sound transmission. 4. Execution & Quality Control: Partition walls must be installed strictly squared and leveled, Floor and ceiling tracks must be securely aligned prior to board installation. | m² | 70 |
| 2.2 | False Ceilings | Supply and install plastic suspended false ceiling 60x60 including supporting grid system, hangers, trims, and all necessary accessories, complete in all respects, where all metallic framing is in galvanized steel | m² | 181 |
| Division 3 |  |  |  |  |
| 3 | Finishing |  |  |  |
| 3.1 | Painting for existing walls | Supply and apply acrilyc based paint to all existing walls, with half high rubber paint skirting, including: Light sanding where required to ensure proper adhesion. Local application of wall putty only where necessary to repair surface imperfections, cracks, holes, or uneven areas, including sanding to achieve a smooth finish. Application of additional finishing layers of high-quality anti-graffiti oil-based paint, minimum 3 coats or as required by the Engineer, in white or approved neutral color, ensuring uniform coverage. (First coat as undercoat) Protection of floors and adjacent surfaces by covering the complete area with protective sheets prior to painting works. Including all required materials, tools, equipment, and skilled manpower to complete the works to the Engineer’s satisfaction. | m² | 450 |
|  | Painting for Gypsum new walls | Supply and apply acrilyc based paint to all new walls, with half high rubber paint skirting, including: Light sanding where required to ensure proper adhesion. Local application of wall putty only where necessary to repair surface imperfections or uneven areas, including sanding to achieve a smooth finish. Application of additional finishing layers of high-quality anti-graffiti oil-based paint, minimum 3 coats or as required by the Engineer, in white or approved neutral color, ensuring uniform coverage. (First coat as undercoat) Protection of floors and adjacent surfaces by covering the complete area with protective sheets prior to painting works. Including all required materials, tools, equipment, and skilled manpower to complete the works to the Engineer’s satisfaction. | m² | 250 |
| Division 4 |  |  |  |  |
| 4 | Sanitary |  |  |  |
| 4.1 | Sanitary fixtures | Supply and installation of a wall mounted porcelain washbasins oval/rectangular shape (50 cm) (made in Europe) of suitable height to be determined on site (European manufacturer) including: [b] water connection (PPR, concealed) & sewer connection (PVC, concealed) [c] cast iron anti-corrosion painted brackets [d] all necessary materials, accessories , requirements and manpower to make it properly serviceable mixers, siphons, valves, connections to water supply and drainage networks, and all necessary accessories, complete in all respects. Supply and installation of a wall mounted porcelain washbasins oval/rectangular shape (50 cm) (made in Europe) of suitable height to be determined on site (European manufacturer) including: [a] stainless steel P-trap high quality | No. | 8 |
| 4.2 | Vanity top (Kitchen) | Supply and Install countertop & bassin for kitchen including in premium natural granite slab (3cm thickness), polished with honed edges (standard), with cutout for sink as described below. Edge Profile: 4cm half bullnose Color: Client's selection from approved samples Treatment: Stain-resistant sealant application (2 coats) -Stainless steel, undermount, single-bowl bassin (50×45cm, 18-gauge, satin finish) Mixer tap with a Single-lever pull-down sprayer and Ceramic cartridge (drip-free) with all accessories including but may not be limited to : Sink strainer with overflow protection Angle valves (chrome-plated brass). Color and design as approved by engineer | No. | 1 |
| Division 5 |  |  |  |  |
| 5 | Aluminum Works |  |  |  |
| 5.1 | Aluminum Doors | Supply and install aluminum SIDEM 2000 profile doors (w 90cm, h200cm) with 6mm thick single glass panels, including frames, hinges, locks, handles, sealing, and all necessary accessories, complete as per approved shop drawings, with a matching sliding insect screen with a multi-rail sliding system engineered for smooth, silent, and low-friction track operation. | No. | 11 |
| 5.3 | Aluminum Partitions | Supply and install aluminum SIDEM 2000 profile partition of 3m height, consisting of 1.50 m aluminum sheet at bottom and 1.50 m 6mm glass above, including framing system, fixing, sealing, and all necessary accessories, complete in all respects. | m² | 15 |
| 5.4 | Aluminum Windows | Supply and install aluminum SIDEM 2000 profile windows (1m x 2m) with 6mm thick double glass panels, including frames, hinges, locks, handles, sealing, and all necessary accessories, complete as per approved shop drawings, with a matching sliding insect screen with a multi-rail sliding system engineered for smooth, silent, and low-friction track operation. | m² | 19 |
| 5.5 | Repair and reuse existing aluminum | Dismantling, repairing and relocating 2 existing aluminum partitions; including all accecerries if needed (new location as per drawing) | m² | 24 |
| Division 6 |  |  |  |  |
| 6 | Flooring |  |  |  |
| 5.1 | Floor Polishing | Clean and Rub the Existing Floor Tiles and Skirting Rehabilitate any type of flooring tiles and skirting (marbels, mosaic, etc...) works includes all materisals and labors and the following steps: - Preparing and saftty - Cleaning and structural repairing - Grinding and leveling - Progressive honing - High Gloss Polishing - Penetration sealing. A trial mockup (one by type of floor tiles) will be performed by the contractor prior to proceed with the complete item. Proposed location and results should be pproved by the client. | m² | 300 |
| Division 7 |  |  |  |  |
| 7 | HVAC Works |  |  |  |
| 7.1 | Split AC Units, 9000 BTU | Supply & Installation of 9,000 BTU Inverter Split AC Units 220V, 50Hz, non-flammable refrigerant (R32/R290) Includes 3m copper piping (Ø6.35mm + Ø9.52mm) with insulation Complete installation with electrical wiring, drainage, and mounting System. Including testing & commissioning (vacuum and pressure testing refrigerant charging and performance verification | No. | 7 |
| 7.2 | Split AC Units, 18000 BTU | Supply & Installation of 18,000 BTU Inverter Split AC Units 220V, 50Hz, non-flammable refrigerant (R32/R290) Includes 3m copper piping (Ø6.35mm + Ø12.70mm) with insulation Complete installation with electrical wiring, drainage, and mounting System. Including testing & commissioning (vacuum and pressure testing refrigerant charging and performance verification | No. | 1 |
| Division 8 |  |  |  |  |
| 8 | Furniture |  |  |  |
| 8.1 | Benche | Supply and install waiting room benches in upholsted galvanized steel, complete with all necessary accessories and fixing. | No. | 3 |
| Division 9 |  |  |  |  |
| 9 | Dental Clinic |  |  |  |
| 9.1 | Wooden raised Platform | Supply and install removable wooden raised floor system in dental clinic areas, including supporting structure, panels, finishing, openings where required, and all necessary accessories, complete in all respects. The system shall allow future accessibility and flexibility for installation, maintenance, and modification of dental clinic infrastructure and equipment. | m² | 24 |
| 9.2 | Specialized works | Preparation and execution of all required infrastructure for dental clinic, including but not limited to drainage, water supply, electrical outlets, medical equipment connections, and all related works, complete in all respects (as per attached drawings and requirements). | LS | 1 |
| Division 10 |  |  |  |  |
| 10 | Electrical Works |  |  |  |
| 10.1 | Lighting Points (60*60) | Supply and install all internal electrical works including LED lighting fixtures 30 (60cm*60cm), switches 20, sockets 50, wiring, Data 11,conduits,Cabinet 1 ,Electrical panel 2,cicuit breaker 24,distribution, and all necessary accessories, complete as per specifications and drawings. | LS | 1 |
| 10.2 | CCTV System | Supply and install indoor and outdoor CCTV system 10 cameras, cabling, recording unit, accessories, and all necessary components for proper operation, complete with testing and commissioning. | LS | 1 |
| Division 11 |  |  |  |  |
| 11 | Wooden Elements |  |  |  |
| 11.1 | Existing wooden windows | Repair and rehabilitation of 18 existing external wooden windows and doors, including sanding, fixing, replacement of damaged parts if needed, and finishing, complete in all respects. | U | 18 |