## Annex 2 – Bid Form completed signed and stamped

- All bids must be typed; handwritten bids will not be considered.

- The contract will be awarded to the lowest-cost technically compliant complete lot or complete bid.

- The quantities mentioned below are required unless there is a variation within a specified range with proper justification. The LRC reserves the right to increase or decrease the quantities.

- The bidder is not permitted to alter the section labelled “LRC to complete” below. Bidder is not allowed to use any other template.

- In the event of any discrepancy between the unit price and the total price, LRC reserves the right to rely on the unit price as stated in the financial bid, rather than the total price without go back to the supplier. The unit price must be provided with no more than two decimal places.

- Only one option is allowed for biding

- The bidder shall price each of the three distribution-line schedules separately. All unit rates shall include labor, materials, equipment, transport, testing, permits, overheads, incidental works, and all obligations necessary for the complete execution and handover of the works.

LOT 1:  OMAR AL FAROKH SCHOOL -CERD: 40 - BEIRUT

| LRC To Complete |  |  |  | Bidder To Complete |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT | VAT (%) | Unit Price in USD, inclusive VAT | Total Price in USD, inclusive VAT |
| 1.1 | Demolition | LS | 1 |  |  |  |  |
| 1.2 | Concrete block wall — 12 cm | m² | 4 |  |  |  |  |
| 1.3 | Internal Plastering | m² | 70 |  |  |  |  |
| 1.4 | Floor tiles (anti-slip ceramic) | m² | 6 |  |  |  |  |
| 1.5 | Wall tiles 20×60 | m² | 55 |  |  |  |  |
| 1.6 | Ceiling & wall paint (with putty) | m² | 15 |  |  |  |  |
| 1.7 | Wooden door with frame (approx. 0.90 × 2.10 m) | No. | 4 |  |  |  |  |
| 1.8 | Soap dispenser | No. | 3 |  |  |  |  |
| 1.9 | Mirror | No. | 3 |  |  |  |  |
| 1.10 | Toilet paper holder | No. | 1 |  |  |  |  |
| 1.11 | Plexi sign board — wall-fixed (600 × 400 mm) | No. | 4 |  |  |  |  |
| 1.12 | Shelf under mirror | No. | 2 |  |  |  |  |
| 1.13 | Lighting point — concealed (LED 40 W) | No. | 7 |  |  |  |  |
| 1.14 | Toilet exhaust fan | No. | 4 |  |  |  |  |
| 1.16 | Circuit breaker box (6-way, double-pole devices) | No. | 3 |  |  |  |  |
| 1.16 | Wall-hung WC | No. | 1 |  |  |  |  |
| 1.17 | Hand-held bidet (shattaf) | No. | 1 |  |  |  |  |
| 1.18 | Sanitary napkin pedal bin (MHM) | No. | 1 |  |  |  |  |
| 1.19 | Paper-towel dispenser | No. | 3 |  |  |  |  |
| 1.20 | Kitchen sink – granite (with cabinet) | No. | 3 |  |  |  |  |
| 1.21 | Wash basin – granite round 50 cm (supply & install only) | No. | 3 |  |  |  |  |
| 1.22 | Water mixer – wash basin / sink | No. | 3 |  |  |  |  |
| 1.23 | Short push/self-close tap (basin) | No. | 3 |  |  |  |  |
| 1.24 | Stainless-steel P-trap | No. | 3 |  |  |  |  |
| 1.25 | PPR pipe ½″ (concealed) | m | 20 |  |  |  |  |
| 1.26 | PVC pipe 2″ | m | 14 |  |  |  |  |
| 1.27 | Floor drain 15×15 (stainless) | No. | 3 |  |  |  |  |
| 1.28 | Cleanout 20×20 (stainless) | No. | 2 |  |  |  |  |
| 1.29 | Toilet ventilation (in-line duct fan) | No. | 4 |  |  |  |  |
| 1.30 | Pressure test | LS | 2 |  |  |  |  |
| 1.31 | Flow switch | No. | 1 |  |  |  |  |
|  | Total Amount including VAT for Lot 1 |  |  |  |  |  |  |
|  | Lead time of Delivery |  |  |  |  |  |  |

LOT 2:  RAWDAT ABI HAIDAR SCHOOL- CERD: 20 – BEIRUT

| LRC To Complete |  |  |  | Bidder To Complete |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT | VAT (%) | Unit Price in USD, inclusive VAT | Total Price in USD, inclusive VAT |
| 1.1 | RC slab — 25 MPa (T14, 125 kg/m³) | m³ | 2 |  |  |  |  |
| 1.2 | Demolition | Ls | 1 |  |  |  |  |
| 1.3 | Excavation & Backfilling | m3 | 6 |  |  |  |  |
| 1.4 | Sewer / drainage network jetting per manhole run | U | 1 |  |  |  |  |
| 1.5 | Asphalt layer (new subbase + base + asphalt) | m3 | 5 |  |  |  |  |
| 1.6 | Subbase material | m3 | 10 |  |  |  |  |
| 1.7 | PVC pipe 6″ | m | 12 |  |  |  |  |
| 1.8 | Manhole 100×100×120 cm | No. | 2 |  |  |  |  |
|  | Total Amount including VAT for Lot 2 |  |  |  |  |  |  |
|  | Lead time of Delivery |  |  |  |  |  |  |

LOT 3:  MARJ EL ZOHOR SCHOOL -CERD: 1187 - HASBAYA

| LRC To Complete |  |  |  | Bidder To Complete |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT | VAT (%) | Unit Price in USD, inclusive VAT | Total Price in USD, inclusive VAT |
| 1.1 | DEMOLITION | m² | 100 |  |  |  |  |
| 1.2 | Concrete block wall — 12 cm | m² | 10 |  |  |  |  |
| 1.3 | Internal Plastering | m² | 130 |  |  |  |  |
| 1.4 | Floor tiles (anti-slip ceramic) | m² | 35 |  |  |  |  |
| 1.5 | Wall tiles 20×60 | m² | 70 |  |  |  |  |
| 1.6 | Ceiling & wall paint (with putty) | m² | 35 |  |  |  |  |
| 1.7 | Wooden door (with frame) | No. | 5 |  |  |  |  |
| 1.8 | Steel window — full repair (all hardware) | No. | 6 |  |  |  |  |
| 1.9 | Soap dispenser | No. | 3 |  |  |  |  |
| 1.10 | Mirror | No. | 3 |  |  |  |  |
| 1.11 | Toilet paper holder | No. | 4 |  |  |  |  |
| 1.12 | Plexi sign board — wall-fixed (600 × 400 mm) | No. | 2 |  |  |  |  |
| 1.13 | Shelf under mirror | No. | 3 |  |  |  |  |
| 1.14 | Lighting point — concealed (LED 40 W) | No. | 10 |  |  |  |  |
| 1.15 | Toilet exhaust fan | No. | 4 |  |  |  |  |
| 1.16 | Western sitting WC (porcelain) | No. | 4 |  |  |  |  |
| 1.17 | Hand-held bidet (shattaf) | No. | 4 |  |  |  |  |
| 1.18 | Paper-towel dispenser | No. | 4 |  |  |  |  |
| 1.19 | Kitchen sink – granite (with cabinet) | No. | 1 |  |  |  |  |
| 1.2 | Wash basin – ceramic (supply & install only) | No. | 3 |  |  |  |  |
| 1.21 | Water mixer – wash basin / sink | No. | 1 |  |  |  |  |
| 1.22 | Water mixer – shower | No. | 2 |  |  |  |  |
| 1.23 | Short push/self-close tap (basin) | No. | 3 |  |  |  |  |
| 1.24 | PPR pipe 1″ (concealed) | m | 20 |  |  |  |  |
| 1.25 | PPR pipe ½″ (concealed) | m | 30 |  |  |  |  |
| 1.26 | PVC pipe 2″ | m | 20 |  |  |  |  |
| 1.27 | PVC pipe 4″ | m | 20 |  |  |  |  |
| 1.28 | PEX pipe ½″ (16 mm) — concealed | m | 10 |  |  |  |  |
| 1.29 | Floor drain 15×15 (stainless) | No. | 7 |  |  |  |  |
| 1.30 | Cleanout 20×20 (stainless) | No. | 2 |  |  |  |  |
| 1.31 | Manhole 60×60×100 cm | No. | 1 |  |  |  |  |
| 1.32 | Gas water heater (instantaneous, LPG) — with flue | No. | 1 |  |  |  |  |
| 1.33 | Gas jar (LPG cylinder) for gas water heater — any capacity, measured per kg | kg | 12 |  |  |  |  |
| 1.34- | Flow switch | No. | 2 |  |  |  |  |
| 1.35 | Pressure test | LS | 2 |  |  |  |  |
| 1.36 | Water tank 2000 L (PE, 3-layer) | No. | 2 |  |  |  |  |
|  | Total Amount including VAT for Lot 3 |  |  |  |  |  |  |
|  | Lead time of Delivery |  |  |  |  |  |  |

LOT 4:  AKKAR SHELTERS / SCHOOLS- PUBLIC BUILDINGS- MUNICIPALITY

| LRC To Complete |  |  |  | Bidder To Complete |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item number | Item/Milestone Required | Required UOM | Estimated Qty | Unit Price in USD, Exclusive VAT | VAT (%) | Unit Price in USD, inclusive VAT | Total Price in USD, inclusive VAT |
| 1.1 | DEMOLITION | Unit | 3 |  |  |  |  |
| 1.2 | Wooden door — full repair (all components) | No. | 20 |  |  |  |  |
| 1.3 | European lock + handle | No. | 10 |  |  |  |  |
| 1.4 | New steel window (approx. 1.20 × 1.00 m) | No. | 5 |  |  |  |  |
| 1.5 | Steel window — full repair (all hardware) | No. | 10 |  |  |  |  |
| 1.6 | Aluminum door/window — full repair (all hardware) | No. | 20 |  |  |  |  |
| 1.7 | Soap dispenser | No. | 15 |  |  |  |  |
| 1.8 | Mirror | No. | 10 |  |  |  |  |
| 1.9 | Toilet paper holder | No. | 30 |  |  |  |  |
| 1.1 | Lighting point — concealed (LED 40 W) | No. | 20 |  |  |  |  |
| 1.11 | Solar water boiler 300 L — thermal collector type | No. | 2 |  |  |  |  |
| 1.12 | Toilet exhaust fan | No. | 20 |  |  |  |  |
| 1.13 | Western sitting WC (porcelain) | No. | 10 |  |  |  |  |
| 1.14 | Repair Western (floor-standing) WC — complete with all accessories | No. | 20 |  |  |  |  |
| 1.15 | Hand-held bidet (shattaf) | No. | 30 |  |  |  |  |
| 1.16 | Repair single-basin sink — complete with P-trap, tap and all accessories | No. | 20 |  |  |  |  |
| 1.17 | Wash basin – ceramic (supply & install only) | No. | 10 |  |  |  |  |
| 1.18 | Short push/self-close tap (basin) | No. | 10 |  |  |  |  |
| 1.19 | PPR pipe 1″ (concealed) | m | 80 |  |  |  |  |
| 1.2 | PPR pipe ½″ (concealed) | m | 20 |  |  |  |  |
| 1.21 | Floor drain 15×15 (stainless) | No. | 10 |  |  |  |  |
| 1.22 | Cleanout 20×20 (stainless) | No. | 10 |  |  |  |  |
| 1.23 | Flow switch | No. | 13 |  |  |  |  |
| 1.24 | Pressure test | LS | 10 |  |  |  |  |
| 1.25 | Water tank 2000 L (PE, 3-layer) | No. | 10 |  |  |  |  |
| 1.26 | Water pump ¾ HP (SS) | No. | 3 |  |  |  |  |
|  | Total Amount including VAT for Lot 4 |  |  |  |  |  |  |
|  | Lead time of Delivery |  |  |  |  |  |  |

## ANNEX 3: DETAILED SPECIFICATIONS

As per: 2026-101Terms of Reference

LOT 1:  OMAR AL FAROKH SCHOOL -CERD: 40 - BEIRUT

| Item No. | Item Code | BIM / MasterFormat | Item / Title | Standard | Description (English) | Type | Loc. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | SW-1.01 | 02 41 (Demolition) | Demolition |  | Supply, install, test and hand over Demolition. The works include all cutting, breaking-out and dismantling, loading, haulage and legal disposal of debris at an approved dump, and protection of adjacent works. Priced as a lump sum for the complete works. | demoliton | both |
| 1.2 | AR-2.02 | 04 22 (Concrete unit masonry) | Concrete block wall — 12 cm | EN 771-3 | Supply, install, test and hand over Concrete block wall — 12 cm. The works include laying blocks in 1:4 sand-cement mortar, reinforcement and ties as required, and a plumb, level finish. Measured per square metre of completed work. | Supply & Install | Both |
| 1.3 | AR-4.02 | 09 24 (Cement plaster) | Internal Plastering | EN 998-1 | Supply, install, test and hand over Internal Plastering. The works include removal of loose material, a rush/key coat then two coats of 1:4 mortar (350 kg cement/m³) to 20 mm, and scaffolding as needed. Measured per square metre of completed work. | Supply & Install | Interior |
| 1.4 | AR-5.02 | 09 30 (Tiling) | Floor tiles (anti-slip ceramic) | EN 14411 | Supply, install, test and hand over Floor tiles (anti-slip ceramic). The works include substrate preparation, sand/mortar or adhesive bedding, cross spacers, grouting, PVC edge/coving trims, silicone sealing and slope to floor drains. Measured per square metre of completed work. | Supply & Install | Both |
| 1.5 | AR-5.03 | 09 30 (Tiling) | Wall tiles 20×60 | EN 14411 | Supply, install, test and hand over Wall tiles 20×60. The works include substrate preparation, sand/mortar or adhesive bedding, cross spacers, grouting, PVC edge/coving trims, silicone sealing and slope to floor drains. Measured per square metre of completed work. | Supply & Install | Both |
| 1.6 | AR-6.01 | 09 91 (Painting) | Ceiling & wall paint (with putty) | EN 13300 | Supply, install, test and hand over Ceiling & wall paint (with putty). The works comprise: surface preparation — removal of loose or flaking material, scraping and sanding; ONE coat of alkali-resistant primer/sealer; TWO coats of putty (skim filler) applied and rubbed down between coats to a true, smooth surface; and TWO finishing coats of emulsion paint in the approved colour — i.e. FIVE applied coats in total (1 primer + 2 putty + 2 finish). Includes cutting in, protection of floors, joinery and fittings, scaffolding, and cleaning on completion. Measured per square metre of completed work. | Supply & Install | Both |
| 1.7 | AR-8.01 | 08 14 (Wood doors) | Wooden door with frame (approx. 0.90 × 2.10 m) | EN 14351-1 | Supply, install, test and hand over a wooden door complete with frame, approximate leaf size 0.90 × 2.10 m × 45 mm thick — exact dimensions to be confirmed from site survey before fabrication. The works include a flush plywood/beech-framed leaf faced as specified, a galvanised or hardwood frame 100 × 50 mm anchored to the wall, architraves both sides, three hinges, mortice lockset and lever handles, door stop and threshold, and preparation for painting. Measured per unit, complete and serviceable; where the actual size differs from the stated approximate size by more than 15%, the rate is to be adjusted pro rata on area. | Supply & Install | Interior |
| 1.8 | AR-11.02 | 10 28 (Toilet accessories) | Soap dispenser | ISO 21542 | Supply, install, test and hand over Soap dispenser. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.9 | AR-11.03 | 10 28 (Toilet accessories) | Mirror | ISO 21542 | Supply, install, test and hand over Mirror. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.10 | AR-11.04 | 10 28 (Toilet accessories) | Toilet paper holder | ISO 21542 | Supply, install, test and hand over Toilet paper holder. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.11 | AR-11.06 | 10 14 (Signage) | Plexi sign board — wall-fixed (600 × 400 mm) | ISO 7010 | Supply, install, test and hand over a wall-fixed plexiglass (acrylic) sign board, nominal face size 600 × 400 mm × 5 mm thick, with UV-stable printed or engraved graphics to the approved design. The works include stainless-steel standoff fixings at four corners, wall plugs and anchors suitable for the substrate, protective film removal and cleaning on completion, and setting out at the approved height. No post or concrete base is required. A sample and artwork proof are to be approved before manufacture. Measured per unit, complete and serviceable. | Supply & Install | Both |
| 1.12 | AR-11.08 | 10 28 (Toilet accessories) | Shelf under mirror | ISO 21542 | Supply, install, test and hand over Shelf under mirror. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.13 | EL-2.01 | 26 51 (Interior lighting) | Lighting point — concealed (LED 40 W) | EN 12464-1 / EN 60598 | Supply, install, test and hand over Lighting point — concealed (LED 40 W). The works include luminaire, wiring, back-box, switch and socket, and all items for a complete working point. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.14 | EL-3.03 | 23 34 (Fans) | Toilet exhaust fan | EN 13141 | Supply, install, test and hand over Toilet exhaust fan. The works include wiring, conduit, protection and ducting connection, tested and commissioned. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.16 | EL-3.04 | 26 24 (Panelboards) | Circuit breaker box (6-way, double-pole devices) | IEC 61439 | Supply, install, test and hand over a 6-way circuit breaker box. The works include the enclosure and appropriately-rated breakers, wired, labelled and tested. ALL protective devices on single-phase circuits shall be DOUBLE-POLE (2P), switching both line and neutral, in accordance with the project standard for this section; single-pole devices are not acceptable. Measured per unit, complete and serviceable. | Supply & Install | Both |
| 1.16 | PL-1.03 | 22 42 (Fixtures) | Wall-hung WC | EN 997 / EN 14688 | Supply, install, test and hand over Wall-hung WC. The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.17 | PL-1.04 | 22 42 (Fixtures) | Hand-held bidet (shattaf) | EN 997 / EN 14688 | Supply, install, test and hand over Hand-held bidet (shattaf). The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.18 | PL-1.15 | 10 28 (MHM) | Sanitary napkin pedal bin (MHM) | — | Supply and install a lidded pedal-operated sanitary-waste bin in each female/accessible cubicle for menstrual-hygiene waste. | Supply & Install | Interior |
| 1.19 | PL-1.17 | 10 28 (Hygiene) | Paper-towel dispenser | — | Supply and install a wall-mounted paper-towel dispenser with fixings. | Supply & Install | Interior |
| 1.20 | PL-2.01 | 22 42 (Sinks) | Kitchen sink – granite (with cabinet) | EN 997 / EN 14688 | Supply, install, test and hand over Kitchen sink – granite (with cabinet). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.21 | PL-2.04 | 22 42 (Sinks) | Wash basin – granite round 50 cm (supply & install only) | EN 997 / EN 14688 | Supply and install one wall-mounted round granite wash basin (50 cm, European), including brackets and fixing only. Mixer, trap and connections priced separately. | Component | Interior |
| 1.22 | PL-3.01 | 22 42 (Trim) | Water mixer – wash basin / sink | EN 997 / EN 14688 | Supply, install, test and hand over Water mixer – wash basin / sink. The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.23 | PL-3.04 | 22 42 (Trim) | Short push/self-close tap (basin) | EN 997 / EN 14688 | Supply and install Short push/self-close tap (basin). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Component | Interior |
| 1.24 | PL-5.04 | 22 42 (Components) | Stainless-steel P-trap | EN 997 / EN 14688 | Supply and install Stainless-steel P-trap. The works include all fixings, sealing and connections to leave the part complete and serviceable. Measured per unit, complete and serviceable. | Component | Interior |
| 1.25 | PL-7.01 | 22 11 (Water pipe) | PPR pipe ½″ (concealed) | EN ISO 15874 / EN 809 | Supply, install, test and hand over PPR pipe ½″ (concealed). The works include breaking/chasing and re-concreting after approval, foam thermal insulation on exposed runs, copper connections, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both |
| 1.26 | PL-7.04 | 22 13 (Drain pipe) | PVC pipe 2″ | EN 1401 / EN 1329 | Supply, install, test and hand over PVC pipe 2″. The works include breaking/chasing and re-concreting after approval, ventilation piping, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both |
| 1.27 | PL-8.05 | 22 13 (Floor drains) | Floor drain 15×15 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Floor drain 15×15 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.28 | PL-8.04 | 22 13 (Cleanouts) | Cleanout 20×20 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Cleanout 20×20 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Both |
| 1.29 | PL-10.01 | 23 34 (Fans) | Toilet ventilation (in-line duct fan) | EN 13141 | Supply, install, test and hand over Toilet ventilation (in-line duct fan). The works include ductwork and flexible connections, dampers, isolators, control wiring, and testing and commissioning. Measured per unit, complete and serviceable. | Supply & Install | Interior |
| 1.30 | PL-10.02 | 22 08 (Commissioning) | Pressure test | EN 806-4 | Carry out Pressure test. The works include pressurising the system, checking all joints for leaks and issuing results. Priced as a lump sum for the complete works. | Service | Both |
| 1.31 | PL-3.06 | 22 05 (Valves) | Flow switch | EN 200 / EN 817 | Supply, install, test and hand over Flow switch. The works include concealed hot/cold PPR connections, flexible tails, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Both |

LOT 2:  RAWDAT ABI HAIDAR SCHOOL- CERD: 20 – BEIRUT

| Item No | Item Code | BIM / MasterFormat | Item / Title | Standard | Description (English) | Type | Loc. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | AR-1.01 | 03 30 (Cast-in-place conc.) | RC slab — 25 MPa (T14, 125 kg/m³) | EN 206 / ACI 318 | Supply, install, test and hand over RC slab — 25 MPa (T14, 125 kg/m³). The works include sulphate-resisting cement adjacent to soil, plywood formwork for a fair-faced finish, ribbed reinforcement, mechanical vibration and curing. Measured per cubic metre in place. | Supply & Install | Both | m³ |
| 1.2 | SW-1.01 | 02 41 (Demolition) | Demolition | — | Supply, install, test and hand over Demolition. The works include all cutting, breaking-out and dismantling, loading, haulage and legal disposal of debris at an approved dump, and protection of adjacent works. Priced as a lump sum for the complete works. | Demolition | Both | Ls |
| 1.3 | SW-1.02 / SW-1.03 | 31 20 (Earthwork) | Excavation & Backfilling | — | Supply, execute, test and hand over Excavation works. The works include setting out, excavation in any material to the required lines and levels, shoring/sheet piling and dewatering as required, trimming and levelling of the formation, temporary protection of the open excavation, and loading, haulage and legal disposal of surplus excavated material at an approved dump. Backfilling is measured separately under SW-1.03. Measured per cubic metre of excavation in place. Backfilling: Supply, execute, test and hand over Backfilling works. The works include supply of approved granular backfill material (or re-use of selected excavated material where approved), bedding and surround to services, placing and compaction in layers not exceeding 200 mm to 95–98% MDD, watering as required, field density testing, and making good to the existing surface level. Excavation is measured separately under SW-1.02. Measured per cubic metre of compacted backfill in place. | Supply & Install | Both | m3 |
| 1.4 | SW-1.09 | 33 01 (Sewer cleaning) | Sewer / drainage network jetting per manhole run | — | High-pressure jetting and unblocking of sewer/drainage line between two manholes, including rodding and flushing. | Service | Both | U |
| 1.5 | SW-2.02 | 32 12 (Flexible paving) | Asphalt layer (new subbase + base + asphalt) | EN 13108 | Supply, install, test and hand over Asphalt layer (new subbase + base + asphalt). The works include sub-base and base preparation, prime/tack coat, laying and compaction to 5 cm after rolling, and reinstatement to match existing. Measured per square metre of completed work. | Supply & Install | Exterior | m3 |
| 1.6 | SW-2.04 | 32 11 (Base course) | Subbase material | EN 13285 | Supply, install, test and hand over Subbase material. The works include supply, spreading, grading to line and level, watering and compaction, and all required tests. Measured per square metre of completed work. | Supply & Install | Exterior | m3 |
| 1.7 | PL-7.06 | 22 13 (Drain pipe) | PVC pipe 6″ | EN 1401 / EN 1329 | Supply, install, test and hand over PVC pipe 6″. The works include breaking/chasing and re-concreting after approval, ventilation piping, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.8 | PL-8.03 | 33 44 (Manholes) | Manhole 100×100×120 cm | EN 476 | Supply, install, test and hand over Manhole 100×100×120 cm. The works include block construction with concrete cover and medium-duty steel lid, and internal plastering. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |

LOT 3:  MARJ EL ZOHOR SCHOOL -CERD: 1187 - HASBAYA

| Item No | Item Code | BIM / MasterFormat | Item / Title | Standard | Description (English) | Type | Loc. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | SW-1.01 | Demolition | DEMOLITION |  | Supply, install, test and hand over Demolition. The works include all cutting, breaking-out and dismantling, loading, haulage and legal disposal of debris at an approved dump, and protection of adjacent works. Priced as a lump sum for the complete works. | demoliton | both | m² |
| 1.2 | AR-2.02 | 04 22 (Concrete unit masonry) | Concrete block wall — 12 cm | EN 771-3 | Supply, install, test and hand over Concrete block wall — 12 cm. The works include laying blocks in 1:4 sand-cement mortar, reinforcement and ties as required, and a plumb, level finish. Measured per square metre of completed work. | Supply & Install | Both | m² |
| 1.3 | AR-4.02 | 09 24 (Cement plaster) | Internal Plastering | EN 998-1 | Supply, install, test and hand over Internal Plastering. The works include removal of loose material, a rush/key coat then two coats of 1:4 mortar (350 kg cement/m³) to 20 mm, and scaffolding as needed. Measured per square metre of completed work. | Supply & Install | Interior | m² |
| 1.4 | AR-5.02 | 09 30 (Tiling) | Floor tiles (anti-slip ceramic) | EN 14411 | Supply, install, test and hand over Floor tiles (anti-slip ceramic). The works include substrate preparation, sand/mortar or adhesive bedding, cross spacers, grouting, PVC edge/coving trims, silicone sealing and slope to floor drains. Measured per square metre of completed work. | Supply & Install | Both | m² |
| 1.5 | AR-5.03 | 09 30 (Tiling) | Wall tiles 20×60 | EN 14411 | Supply, install, test and hand over Wall tiles 20×60. The works include substrate preparation, sand/mortar or adhesive bedding, cross spacers, grouting, PVC edge/coving trims, silicone sealing and slope to floor drains. Measured per square metre of completed work. | Supply & Install | Both | m² |
| 1.6 | AR-6.01 | 09 91 (Painting) | Ceiling & wall paint (with putty) | EN 13300 | Supply, install, test and hand over Ceiling & wall paint (with putty). The works comprise: surface preparation — removal of loose or flaking material, scraping and sanding; ONE coat of alkali-resistant primer/sealer; TWO coats of putty (skim filler) applied and rubbed down between coats to a true, smooth surface; and TWO finishing coats of emulsion paint in the approved colour — i.e. FIVE applied coats in total (1 primer + 2 putty + 2 finish). Includes cutting in, protection of floors, joinery and fittings, scaffolding, and cleaning on completion. Measured per square metre of completed work. | Supply & Install | Both | m² |
| 1.7 | AR-8.01 | 08 14 (Wood doors) | Wooden door (with frame) | EN 14351-1 | Supply, install, test and hand over a wooden door complete with frame, approximate leaf size 0.80 × 2.00 m × 45 mm thick — exact dimensions to be confirmed from site survey before fabrication. The works include a flush plywood/beech-framed leaf faced as specified, a galvanised or hardwood frame 100 × 50 mm anchored to the wall, architraves both sides, three hinges, mortice lockset and lever handles, door stop and threshold, and preparation for painting. Measured per unit, complete and serviceable; where the actual size differs from the stated approximate size by more than 15%, the rate is to be adjusted pro rata on area. | Supply & Install | Interior | No. |
| 1.8 | AR-10.04 | 08 01 (Repair) | Steel window — full repair (all hardware) | EN 14351-1 | Full overhaul repair of a steel window: straighten/reweld frame and sash, replace hinges/handles/locks, treat corrosion, prime and repaint, reglaze as needed. Components itemised separately in AR-12. | Repair | Both | No. |
| 1.9 | AR-11.02 | 10 28 (Toilet accessories) | Soap dispenser | ISO 21542 | Supply, install, test and hand over Soap dispenser. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.10 | AR-11.03 | 10 28 (Toilet accessories) | Mirror | ISO 21542 | Supply, install, test and hand over Mirror. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.11 | AR-11.04 | 10 28 (Toilet accessories) | Toilet paper holder | ISO 21542 | Supply, install, test and hand over Toilet paper holder. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.12 | AR-11.05 | 10 14 (Signage) | Plexi sign board — wall-fixed (600 × 400 mm) | ISO 7010 | Supply, install, test and hand over a wall-fixed plexiglass (acrylic) sign board, nominal face size 600 × 400 mm × 5 mm thick, with UV-stable printed or engraved graphics to the approved design. The works include stainless-steel standoff fixings at four corners, wall plugs and anchors suitable for the substrate, protective film removal and cleaning on completion, and setting out at the approved height. No post or concrete base is required. A sample and artwork proof are to be approved before manufacture. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.13 | AR-11.06 | 10 28 (Toilet accessories) | Shelf under mirror | ISO 21542 | Supply, install, test and hand over Shelf under mirror. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.14 | EL-2.01 | 26 51 (Interior lighting) | Lighting point — concealed (LED 40 W) | EN 12464-1 / EN 60598 | Supply, install, test and hand over Lighting point — concealed (LED 40 W). The works include luminaire, wiring, back-box, switch and socket, and all items for a complete working point. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.15 | EL-3.03 | 23 34 (Fans) | Toilet exhaust fan | EN 13141 | Supply, install, test and hand over Toilet exhaust fan. The works include wiring, conduit, protection and ducting connection, tested and commissioned. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.16 | PL-1.02 | 22 42 (Fixtures) | Western sitting WC (porcelain) | EN 997 / EN 14688 | Supply, install, test and hand over Western sitting WC (porcelain). The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.17 | PL-1.04 | 22 42 (Fixtures) | Hand-held bidet (shattaf) | EN 997 / EN 14688 | Supply, install, test and hand over Hand-held bidet (shattaf). The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.18 | PL-1.17 | 10 28 (Hygiene) | Paper-towel dispenser | — | Supply and install a wall-mounted paper-towel dispenser with fixings. | Supply & Install | Interior | No. |
| 1.19 | PL-2.01 | 22 42 (Sinks) | Kitchen sink – granite (with cabinet) | EN 997 / EN 14688 | Supply, install, test and hand over Kitchen sink – granite (with cabinet). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.2 | PL-2.03 | 22 42 (Sinks) | Wash basin – ceramic (supply & install only) | EN 997 / EN 14688 | Supply and install Wash basin – ceramic (supply & install only). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Component | Interior | No. |
| 1.21 | PL-3.01 | 22 42 (Trim) | Water mixer – wash basin / sink | EN 997 / EN 14688 | Supply, install, test and hand over Water mixer – wash basin / sink. The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.22 | PL-3.02 | 22 42 (Trim) | Water mixer – shower | EN 997 / EN 14688 | Supply, install, test and hand over a shower mixer VALVE ONLY. This item covers the mixing valve/tap body and its escutcheon; the shower head, riser rail, flexible hose and wall bracket are NOT included — for a complete shower assembly use PL-3.03. The works include concealed hot/cold PPR connections, flexible tails, sealing, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.23 | PL-3.03 | 22 42 (Trim) | Short push/self-close tap (basin) | EN 997 / EN 14688 | Supply and install Short push/self-close tap (basin). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Component | Interior | No. |
| 1.24 | PL-7.03 | 22 11 (Water pipe) | PPR pipe 1″ (concealed) | EN ISO 15874 / EN 809 | Supply, install, test and hand over PPR pipe 1″ (concealed). The works include breaking/chasing and re-concreting after approval, foam thermal insulation on exposed runs, copper connections, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.25 | PL-7.01 | 22 11 (Water pipe) | PPR pipe ½″ (concealed) | EN ISO 15874 / EN 809 | Supply, install, test and hand over PPR pipe ½″ (concealed). The works include breaking/chasing and re-concreting after approval, foam thermal insulation on exposed runs, copper connections, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.26 | PL-7.04 | 22 13 (Drain pipe) | PVC pipe 2″ | EN 1401 / EN 1329 | Supply, install, test and hand over PVC pipe 2″. The works include breaking/chasing and re-concreting after approval, ventilation piping, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.27 | PL-7.05 | 22 13 (Drain pipe) | PVC pipe 4″ | EN 1401 / EN 1329 | Supply, install, test and hand over PVC pipe 4″. The works include breaking/chasing and re-concreting after approval, ventilation piping, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.28 | PL-7.09 | 22 11 (Water pipe) | PEX pipe ½″ (16 mm) — concealed | EN ISO 15875 | Supply, install, test and hand over cross-linked polyethylene (PEX) water pipe, size ½″ (16 mm), PN10 rated for hot and cold potable water, with an oxygen barrier where connected to a heating circuit. The works include chasing/breaking and re-concreting after approval, laying in protective conduit where concealed in screed or slab, foam thermal insulation on exposed runs, press or compression fittings of the same manufacturer's system, brass manifolds and transition couplings to PPR or copper where required, fixing clips at the specified centres, and pressure testing. Measured per linear metre of pipe installed, fittings deemed included. | Supply & Install | Both | m |
| 1.29 | PL-8.05 | 22 13 (Floor drains) | Floor drain 15×15 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Floor drain 15×15 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.30 | PL-8.04 | 22 13 (Cleanouts) | Cleanout 20×20 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Cleanout 20×20 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.31 | PL-8.03 | 33 44 (Manholes) | Manhole 60×60×100 cm | EN 476 | Supply, install, test and hand over Manhole 60×60×100 cm. The works include block construction with concrete cover and medium-duty steel lid, and internal plastering. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.32 | PL-9.03 | 22 33 (Water heaters — gas) | Gas water heater (instantaneous, LPG) — with flue | EN 26 / EN 30 / EN 1949 | Supply, install, test and hand over an LPG gas water heater. The works include a room-sealed or forced-draught (Type C / fan-flued) instantaneous gas water heater of 10–16 L/min output, with electronic ignition, flame-failure safety device, anti-frost and over-temperature protection; a stainless-steel flue with terminal, wall sleeve and weatherproofing discharging to open air; an LPG regulator, approved flexible or copper gas line with a manual shut-off valve and a leak test on completion; hot and cold water connections with isolating valves and a pressure-relief valve; electrical supply where required; and permanent room ventilation openings to the gas-appliance regulations. Open-flue heaters are NOT permitted in bathrooms or unventilated rooms. Commissioning, combustion check and user handover instruction are included. Gas cylinders are excluded. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.33 | PL-9.04 | 23 11 (Fuel gas storage) | Gas jar (LPG cylinder) for gas water heater — any capacity, measured per kg | EN 1442 / ISO 4706 / EN 16129 | Supply and place a refillable LPG gas jar (gas cylinder) to serve the gas water heater under PL-9.03, complete with its first fill of gas. The cylinder shall be a certified, in-date steel LPG cylinder to EN 1442 / ISO 4706 bearing a legible manufacture and re-test date stamp, fitted with a compliant self-closing valve and protective cap, free of corrosion, dents or weld defects, and hydrostatically re-tested within the required interval. The works include delivery to site, a levelled and ventilated standing position outdoors or in a ventilated cylinder cabinet, a wall chain or strap restraint to prevent toppling, a compatible pressure regulator and approved hose or copper connection with a manual shut-off valve, a leak test with soap solution or a gas detector on completion, and hazard labelling. Storage location must comply with the gas-appliance regulations: not inside bathrooms, stairwells, basements or unventilated rooms, and clear of drains, heat sources and electrical equipment. THIS ITEM IS MEASURED PER KILOGRAM OF NOMINAL GAS CAPACITY, so that any cylinder size can be priced from a single rate — for example a 10 kg jar is measured as 10, a 12.5 kg jar as 12.5, and a 35 kg commercial jar as 35. Enter the total kg required across all cylinders in the quantity column. Subsequent gas refills during operation are excluded and are not part of this contract. | Supply & Install | Exterior | kg |
| 1.34 | PL-3.06 | 22 05 (Valves) | Flow switch | EN 200 / EN 817 | Supply, install, test and hand over Flow switch. The works include concealed hot/cold PPR connections, flexible tails, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.35 | PL-10.02 | 22 08 (Commissioning) | Pressure test | EN 806-4 | Carry out Pressure test. The works include pressurising the system, checking all joints for leaks and issuing results. Priced as a lump sum for the complete works. | Service | Both | LS |
|  | PW-1 WATER STORAGE TANKS |  |  |  |  |  |  |  |
| 1.36 | PW-1.03 | 22 12 (Water tanks) | Water tank 2000 L (PE, 3-layer) | EN 12573 | Supply, install, test and hand over Water tank 2000 L (PE, 3-layer). The works include 3-layer polyethylene tank, all supply/outlet accessories and European valves for inlet and outlet. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |

LOT 4:  AKKAR SHELTERS / SCHOOLS- PUBLIC BUILDINGS- MUNICIPALITY

| Item No | Item Code | BIM / MasterFormat | Item / Title | Standard | Description (English) | Type | Loc. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | SW-1.01 | Demolition | DEMOLITION |  | Supply, install, test and hand over Demolition. The works include all cutting, breaking-out and dismantling, loading, haulage and legal disposal of debris at an approved dump, and protection of adjacent works. Priced as a lump sum for the complete works. | demoliton | both | Unit |
| 1.2 | AR-8.03 | 08 01 (Repair) | Wooden door — full repair (all components) | EN 14351-1 | Full overhaul repair of a wooden door: refix/replace leaf panels and beadings, renew frame joints, replace hinges, lock and handle, apply putty and prepare for painting. Individual components are itemised separately in division AR-12 — use this item only for a complete overhaul. | Repair | Interior | No. |
| 1.3 | AR-8.12 | 08 71 (Door hardware) | European lock + handle | EN 12209 / EN 1935 | Supply, install, test and hand over European lock + handle. The works include all screws, keeps and fixings, adjusted and left fully operable. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.4 | AR-10.02 | 08 51 (Metal windows) | New steel window (approx. 1.20 × 1.00 m) | EN 14351-1 | Supply, install, test and hand over a new steel window, approximate size 1.20 × 1.00 m — exact dimensions to be confirmed from site survey before fabrication. The works include a hot-rolled or pressed steel frame and opening sash in sections not less than 1.5 mm thick, single or double glazing as specified, glazing beads, gaskets and silicone perimeter seal, an insect screen, handles, stays and locking hardware, galvanising or anti-corrosive priming with two finish coats in the approved colour, and making good to the reveal. Measured per unit, complete and serviceable; where the actual size differs from the stated approximate size by more than 15%, the rate is to be adjusted pro rata on area. | Supply & Install | Interior | No. |
| 1.5 | AR-10.04 | 08 01 (Repair) | Steel window — full repair (all hardware) | EN 14351-1 | Full overhaul repair of a steel window: straighten/reweld frame and sash, replace hinges/handles/locks, treat corrosion, prime and repaint, reglaze as needed. Components itemised separately in AR-12. | Repair | Both | No. |
| 1.6 | AR-7.08 | 08 01 (Repair) | Aluminum door/window — full repair (all hardware) | EN 14351-1 | Full overhaul repair of an aluminum door or window: realign and refix leaf/sash, replace worn hinges, rollers, locks and handles, renew gaskets and silicone, and make fully operable. Individual components are also itemised separately in division AR-12 — use this item only for a complete overhaul. | Repair | Both | No. |
| 1.7 | AR-11.02 | 10 28 (Toilet accessories) | Soap dispenser | ISO 21542 | Supply, install, test and hand over Soap dispenser. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.8 | AR-11.03 | 10 28 (Toilet accessories) | Mirror | ISO 21542 | Supply, install, test and hand over Mirror. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.9 | AR-11.04 | 10 28 (Toilet accessories) | Toilet paper holder | ISO 21542 | Supply, install, test and hand over Toilet paper holder. The works include all fixings and brackets, mounted at the correct height, level and secure. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.1 | EL-2.01 | 26 51 (Interior lighting) | Lighting point — concealed (LED 40 W) | EN 12464-1 / EN 60598 | Supply, install, test and hand over Lighting point — concealed (LED 40 W). The works include luminaire, wiring, back-box, switch and socket, and all items for a complete working point. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.11 | EL-6.09 | 48 14 (Solar thermal) | Solar water boiler 300 L — thermal collector type |  | Supply, install, test and commission Solar water boiler 300 L. The works include solar collector for hot climates (≥50 °C), tank, check valve and all connections. Measured per unit, complete and serviceable. | Assembly | Exterior | No. |
| 1.12 | EL-3.03 | 23 34 (Fans) | Toilet exhaust fan | EN 13141 | Supply, install, test and hand over Toilet exhaust fan. The works include wiring, conduit, protection and ducting connection, tested and commissioned. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.13 | PL-1.02 | 22 42 (Fixtures) | Western sitting WC (porcelain) | EN 997 / EN 14688 | Supply, install, test and hand over Western sitting WC (porcelain). The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.14 | PL-1.20 | 22 05 (WC repair) | Repair Western (floor-standing) WC — complete with all accessories | EN 997 / EN 14055 | The Contractor shall inspect every fixture and accessory of the WC and repair or replace any component found faulty, leaking, damaged, missing or not working properly, so that the whole WC is handed over fully functional. Repair and restore to full working order an existing Western floor-standing (close-coupled or low-level) WC, re-using the serviceable pan and cistern. The works include supply and fitting of a new fill (float) valve and flush (outlet) valve set; new push-button or lever handle; new cistern-to-pan rubber gasket (doughnut) and cistern fixing bolts with washers; new pan connector / flexible WC pan manifold to the soil pipe; new floor fixing bolt kit and caps; new chrome angle stop valve and braided flexible hose; new heavy-duty seat cover (European, min 1 cm); re-bedding and sealing of the pan base with sanitary silicone; leak and flush testing. Measured per WC repaired, complete and serviceable. | Repair/Replace | Interior | No. |
| 1.15 | PL-1.04 | 22 42 (Fixtures) | Hand-held bidet (shattaf) | EN 997 / EN 14688 | Supply, install, test and hand over Hand-held bidet (shattaf). The works include triangle chrome stop valve, cistern and complete flush system, seat cover, concealed PPR water and PVC waste connections, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.16 | PL-2.08 | 22 05 (Sink repair) | Repair single-basin sink — complete with P-trap, tap and all accessories | EN 997 / EN 200 / EN 817 | The Contractor shall inspect the complete single-basin sink / wash basin and all its fixtures and accessories, and repair or replace any component found faulty, leaking, damaged, missing or not working properly, so that the whole unit is handed over fully functional. Re-using the serviceable basin, the works include: new or repaired tap / single-lever mixer (including cartridge, aerator and washers); new stainless-steel or chrome P-trap with waste outlet, plug and chain or pop-up, and overflow seals; new trap connector / waste pipe connection to the drain; new chrome angle (triangle) stop valves and braided flexible hoses for hot and cold; replacement of damaged water and waste fittings (elbows, unions, adaptors, rubber seals); checking, re-fixing or replacing the basin brackets / fixings; re-sealing of the basin to the wall or counter with sanitary silicone; and leak and flow testing. Measured per sink repaired, complete and serviceable. Where the basin itself is broken, it is replaced under PL-2.06 instead. | Repair/Replace | Interior | No. |
| 1.17 | PL-2.03 | 22 42 (Sinks) | Wash basin – ceramic (supply & install only) | EN 997 / EN 14688 | Supply and install Wash basin – ceramic (supply & install only). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Component | Interior | No. |
| 1.18 | PL-3.03 | 22 42 (Trim) | Short push/self-close tap (basin) | EN 997 / EN 14688 | Supply and install Short push/self-close tap (basin). The works include brackets/stands, concealed PPR water and PVC waste connections, stainless P-trap, sealing and removal of the defective unit. Measured per unit, complete and serviceable. | Component | Interior | No. |
| 1.19 | PL-7.03 | 22 11 (Water pipe) | PPR pipe 1″ (concealed) | EN ISO 15874 / EN 809 | Supply, install, test and hand over PPR pipe 1″ (concealed). The works include breaking/chasing and re-concreting after approval, foam thermal insulation on exposed runs, copper connections, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.2 | PL-7.01 | 22 11 (Water pipe) | PPR pipe ½″ (concealed) | EN ISO 15874 / EN 809 | Supply, install, test and hand over PPR pipe ½″ (concealed). The works include breaking/chasing and re-concreting after approval, foam thermal insulation on exposed runs, copper connections, and all joints, elbows and sockets. Measured per linear metre completed. | Supply & Install | Both | m |
| 1.21 | PL-8.05 | 22 13 (Floor drains) | Floor drain 15×15 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Floor drain 15×15 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Interior | No. |
| 1.22 | PL-8.04 | 22 13 (Cleanouts) | Cleanout 20×20 (stainless) | EN 1401 / EN 1329 | Supply, install, test and hand over Cleanout 20×20 (stainless). The works include secure, watertight and corrosion-resistant fixing to the drainage line. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.23 | PL-3.06 | 22 05 (Valves) | Flow switch | EN 200 / EN 817 | Supply, install, test and hand over Flow switch. The works include concealed hot/cold PPR connections, flexible tails, and removal of the defective unit. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.24 | PL-10.02 | 22 08 (Commissioning) | Pressure test | EN 806-4 | Carry out Pressure test. The works include pressurising the system, checking all joints for leaks and issuing results. Priced as a lump sum for the complete works. | Service | Both | LS |
|  | PW-1 WATER STORAGE TANKS |  |  |  |  |  |  |  |
| 1.25 | PW-1.03 | 22 12 (Water tanks) | Water tank 2000 L (PE, 3-layer) | EN 12573 | Supply, install, test and hand over Water tank 2000 L (PE, 3-layer). The works include 3-layer polyethylene tank, all supply/outlet accessories and European valves for inlet and outlet. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |
| 1.26 | PW-2.02 | 22 11 (Pumps) | Water pump ¾ HP (SS) | EN ISO 15874 / EN 809 | Supply, install, test and hand over Water pump ¾ HP (SS). The works include stainless-steel construction, automatic switch and pressure control, control panel and all accessories, with head/flow confirmed on site. Measured per unit, complete and serviceable. | Supply & Install | Both | No. |